| Questionnaire Title | Core Questions (Core IRRS modules) |
| --- | --- |
| Module Name | 10. Basic Primary responsibilities of the regulatory body (RB) in emergency |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What authority and responsibilities have been provided by the government to the regulatory body (RB) to regulate on-site emergency preparedness and response (EPR) of facilities or activities that could necessitate emergency response? [Topic 10.1] | B | F |
| Expectation | [Note: This question is only to reveal any specific information related to regulatory authorization as it applies to EPR. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The RB has the relevant authority and responsibilities to regulate the on-site arrangements for preparedness and response for a nuclear or radiological emergency for facilities and activities that could necessitate emergency response actions. If the regulatory authority is shared among different organizations (e.g. nuclear safety authority, health protection agencies, emergency management organizations, etc.), the regulatory authority and responsibilities are clearly and unambiguously assigned between them in relation to various aspects or in relation to various facilities or activities (e.g. nuclear facilities versus transport of radioactive material or use of radioactive sources in industry and medicine), and the regulatory functions are effectively coordinated. The RB has the responsibilities for: - Establishing regulations and guides for EPR for facilities or activities that could necessitate emergency response actions; - Verifying compliance of the on-site emergency arrangements of operating organizations against the regulatory requirements before commencement of operation of the facility or before the conduct of the activity, and afterwards during the lifetime of the facility or the conduct of the activity; - Evaluating some of exercises conducted by the operating organizations; and - Ensuring that the operator’s emergency arrangements are coordinated with those of other organizations and integrated with contingency plans and security plans established for nuclear security purposes. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 33 | 2.21 – 2.24 |
| Safety Standards Series No. GSR Part 7 |  | 40 | 4.11, 6.2 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What are the coordination arrangements are in place between organizations that have authorities and responsibilities in regulating EPR of operating organizations or that may be impacted by the relevant judgements and decisions? [Topic 10.1] | T | T |
| QData |  |  |  |
| Expectation | Authorities and responsibilities for regulating the EPR arrangements of the operating organizations are clearly and unambiguously assigned to one or more relevant organizations. In case that there is more than one organization that regulates various on-site emergency arrangements of operating organizations (e.g. nuclear safety authorities, health protection agencies, emergency management organizations) or of various facilities or activities (e.g. nuclear facilities versus transport of radioactive material or use of radioactive sources in industry and medicine), they coordinate their regulatory functions to ensure consistency among respective arrangements and to avoid overlaps and gaps in regulating on-site EPR. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | In line with the primary question, the coordination among various organizations with authorities and responsibilities to regulate EPR of operating organizations is related to the following areas: - Establishing regulations and guides for EPR for operating organizations of regulated facilities or activities that could necessitate emergency response actions; - Verifying compliance of the on-site emergency arrangements of operating organizations against the regulatory requirements before commencement of operation of the facility or before the conduct of the activity, and afterwards during the lifetime of the facility or the conduct of the activity through: - Review and assessment of the documentation elaborating operator’s emergency arrangements during the licensing process; - Approval of the operator’s emergency plan; - Inspections on EPR arrangements of operating organizations; - Evaluating some of exercises conducted by the operating organizations; and - Ensuring that the operator’s emergency arrangements are coordinated with those of other organizations and integrated with contingency plans and security plans established for nuclear security purposes. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Review and assessment of on-site emergency arrangements. | 51 | Req. 27; 4.45(10) |
| Safety Standards Series No. GSR Part 7 | Regulatory process to cover EPR. | 40 | 4.11 - 4.14, 6.30 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What resources and organizational structure does the regulatory body have to fulfil its responsibilities in regulating the EPR of operating organizations effectively on the basis of a graded approach? [Topic 10.1] | T | T |
| QData |  |  |  |
| Expectation | [Note: This question is only to reveal any specific information related to regulatory authorization as it applies to EPR. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The RB has structured its organization, managed its resources, and has a sufficient number or qualified and competent staff to discharge its authority and fulfil respective responsibilities in regulating the EPR of operating organizations effectively. In doing so, the RB takes account of the hazards and potential consequences of an emergency associated with the facilities and activities under the responsibility of an operating organization. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The resources and organizational structure of various organizations to discharge their authority and to fulfil their responsibilities in regulating the EPR of operating organizations should be determined in consideration of a graded approach. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 7 | Regulatory process to cover EPR. | 39 | 4.8, 4.11, 4.18 |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Organizational structure of the RB and a graded approach. | 41 | Req. 16; Req. 28 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What regulations and guides have been established or adopted by the regulatory body to set up principles, requirements and associated criteria for EPR for the operating organization upon which its regulatory judgements, decisions and actions are based? [Topic 10.2] | B | F |
| Expectation | The RB has established or adopted relevant regulations and issued guides in relation to the on-site emergency arrangements. These regulations and guides require, at least, the operating organization to: - Perform a hazard assessment as a basis for a graded approach in establishing the on-site EPR; - Execute promptly and manages safely and effectively the on-site emergency response including the transition from normal operations to operations under emergency conditions; - Classify promptly the emergency, declares the emergency class, initiates the on-site emergency response and notifies and provides sufficient information to the off-site notification point; - Decide on and takes mitigatory actions on-site; - Assess and determine, at preparedness stage, when and under what conditions assistance from off-site emergency services may be needed to be provided on the site; - Assess the hazards and possible development of hazardous conditions initially and throughout the emergency and take necessary urgent protective actions to protect all persons present at the site in an emergency; - Ensure suitable, reliable and diverse means of communication for use in taking protective actions on the site and for communication with relevant off-site officials; - Protect emergency workers responding on the site; - Communicate with the public effectively in a nuclear or radiological emergency and consistently with relevant off-site response organizations; - Manage radioactive waste generated in an emergency safely and effectively; - Set conditions, criteria and objectives to be met to terminate the emergency on the site and to provide relevant information in this regard to relevant off-site officials; - Document, protect and preserve, to the extent practicable, data and information important for an analysis of the emergency and emergency response; - Analyse the emergency and the emergency response with the aim to identify actions to be taken to avoid other emergencies and to improve emergency arrangements; - Develop capability to be able to effectively respond in an emergency which includes: - Clear assign authorities and responsibilities in EPR for various positions within its organizational structure; - Ensure adequate number of suitably qualified personnel is available to promptly staff necessary positions in an emergency response as well as in long term if needed; - Ensure means for coordination with other organizations in the use of necessary tools, procedures or criteria with the aim to ensure consistency in any relevant assessments made in a nuclear or radiological emergency; - Develop emergency plan, procedures and analytical tools for emergency response which are coordinated with those of other organizations and with other relevant plans; - Validate various procedures and analytical tools through testing them under simulated emergency conditions prior to initial use; - Ensure adequate tools, instruments, supplies, equipment, emergency response facilities and documentation are available to support the on-site emergency response including, where appropriate, alternative supplies for taking on-site mitigatory actions (such as alternative electrical power supply); - Develop and implementing training and exercise programmes; - Establish a quality management programme, as part of its management system, to ensure the availability and reliability of all supplies, equipment, communication systems and facilities, plans, procedures and other arrangements necessary for EPR which includes review and revisions of emergency plans, procedures and other arrangements on a regular basis and maintaining records. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Main legislative acts, regulations and guides related to on-site EPR should be described briefly. The aim is to assess the adequacy of on-site EPR provisions in these regulations and guides for regulating the on-site EPR rather than to make any assessment on the adequacy of the established on-site emergency arrangements. Detailed descriptions on the on-site emergency arrangements required or in place, such as the emergency classification system used by the operator, are not intended to be provided. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 7 | Regulations and guides on EPR. | 40 | 4.12 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the regulatory body ensure that the operating organization is given sufficient authority to promptly take necessary actions on the site to mitigate the consequences of an emergency even if they could result in off-site consequences? [Topic 10.2] | T | T |
| QData |  |  |  |
| Expectation | The RB verifies, at least, at the preparedness stage when the RB reviews the site’s emergency plan that the operating organizations has sufficient authority to promptly take necessary protective actions on the site in response to a nuclear or radiological emergency that could result in off-site consequences. Answer should provide descriptive, factual responses to the question and all necessary documentary evidence to support the responses should be appended. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The necessary protective actions in response to an emergency that could result in off-site consequences should be discussed at the preparedness stage when the RB reviews the operator’s emergency plan. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 7 |  | 41 | 4.15, 5.23 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the regulatory body ensure that the operating organization reviews and, as necessary, revises the emergency arrangements (a) prior to any changes in the facility or activity that affect the existing hazard assessment and (b) when new information becomes available that provides insights into the adequacy of the existing arrangements? [Topic 10.2] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body has established clear provisions to require the operating organizations to review and, as appropriate, to revise their emergency arrangements prior to any changes in the facility or activity that affect the existing hazard assessment and when new information becomes available that provides insights into the adequacy of the existing arrangements. Answer should provide descriptive, factual responses to the question and all necessary documentary evidence to support the responses should be appended. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | These provisions can be set up as regulatory requirements or license conditions. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 47 | 4.31 |
| Safety Standards Series No. GSR Part 7 |  | 46 | 4.26 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the regulatory body verify emergency arrangements of operating organizations are in place and provide sufficient assurance for an effective on-site emergency response for any regulated facility or activity that could necessitate emergency response actions? [Topic 10.3] | A | F |
| Expectation | [NOTE: This question is only to reveal any specific information related to regulatory verification as it applies to EPR. Do not repeat information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The regulatory body verifies the compliance of the on-site emergency arrangements of operating organizations against the regulatory requirements before commencement of operation of the facility or before the conduct of the activity, and afterwards during the lifetime of the facility or the conduct of the activity through: - The review and assessment of emergency plans submitted by operating organizations as part of their license applications; - The approval process of emergency plans of operating organizations which provide all the on-site emergency arrangements in place; - The regulatory inspections on various aspects of EPR; - The observation and evaluation of some emergency exercises conducted by the operating organization to test on-site arrangements to provide, to the extent practicable, assurance of an effective response to a nuclear or radiological emergency. All of the regulatory verifications are conducted in accordance with a graded approach. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 46 | Requirement 23; 25; 27; 31 |
| Safety Standards Series No. GSR Part 7 |  | 44 | 4.11, 4.13, 4.14 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the regulatory body evaluate of the exercises performed by operating organizations to test their emergency arrangements in place? [Topic 10.3] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body observes some exercises conducted by the operating organizations for evaluating their on-site emergency arrangements. These observations are planned to ensure such arrangements are evaluated regularly and in accordance with a graded approach. The RB has internal methodology and criteria for conducting such evaluations against the regulatory requirements. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Regulatory body used any relevant internal documents to evaluate any exercise, including evaluation reports from past exercises documenting the findings, corrective actions requested to be implemented in EPR area etc. in order to ensure compliance with Requirements. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 7 | Verification of compliance. | 40 | 4.11, 4.13, 6.30 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the regulatory body ensure that the emergency arrangements of operating organizations are integrated with those of relevant off-site response organizations and with other plans (such as security plans and contingency plans established for nuclear security purposes)? [Topic 10.3] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body verifies the integration of emergency arrangements with relevant off-site response arrangements and with other plans where appropriate mainly through the review and assessment of the emergency plan, on-site inspections and the evaluation of some exercises through which established coordination arrangements are witnessed and tested, as appropriate. The regulatory body has established consultation provisions with other organizations (or among various internal departments and sections having relevant responsibilities) through which the satisfactory fulfilment of respective requirements for coordination and integration is confirmed. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Answer should provide descriptive, factual responses to the question and all necessary documentary evidence to support the responses should be appended. Thus, written records of any such consultation and established protocols and their use in the past can be used to demonstrate compliance with this requirement. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Review and assessment. | 51 | Requirement 27; 4.45 (10); 4.50 |
| Safety Standards Series No. GSR Part 7 | Integration of emergency arrangements with those of other organizations and with contingency and security plans. | 40 | 4.11, 4.13, 4.14, 6.30 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What responsibilities in the event of a nuclear or radiological emergency has the Government assigned to the regulatory body? [Topic 10.4] | A | F |
| Expectation | The regulatory body is part of the national EPR framework and has clearly allocated roles and responsibilities for preparedness and response for a nuclear or radiological emergency. Answer should provide descriptive, factual responses to the question and all necessary documentary evidence to support the responses should be appended. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Example of roles and responsibilities for preparedness and response for a nuclear or radiological emergency may include providing advice to the government and response organizations, and providing expert services, as appropriate (e.g. services for radiation monitoring and risk assessment for actual and expected future radiation risks). These roles and responsibilities are expected to be clearly specified in the legislative framework and elaborated in the regulatory body’s emergency plan. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 33 | 2.24 |
| Safety Standards Series No. GSR Part 7 |  | 39 | 4.7 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the regulatory body ensure that it has and maintains an adequate capability to fulfil its roles and responsibilities in response to a nuclear or radiological emergency? [Topic 10.4] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body maintains its own emergency plan, procedures and other arrangements under its management system in order to effectively execute expected emergency response role and responsibilities to include: adequate emergency organization structure and staffing plan with clearly assigned authorities and responsibilities; facilities, equipment, tools, communication systems etc. The regulatory body has established and implements internal programmes for training, drills and exercises. Answer should provide descriptive, factual responses to the question and all necessary documentary evidence to support the responses should be appended. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 7 | Need for each response organization to develop necessary procedures and tools. | 79 | 6.5, 6.9, 6.12, 6.13, 6.17, 6.20 – 6.22, 6.28, 6.30, 6.34 |