| Questionnaire Title | Facilities and Activities and Exposure Situations |
| --- | --- |
| Module Name | Regulation of Fuel Cycle Facilities |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are provisions in place for the authorization (licensing) of fuel cycle facilities during the different phases of their life cycle? [Topic 5.4] | A | F |
| Expectation | [NOTE: This primary question is only to reveal any specific information related to regulatory authorization as it applies to fuel cycle facilities. DO NOT repeat information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] Specific provisions are in place for authorization (licensing) of fuel cycle facilities during the different stages of their life cycle. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Different types of authorization are required to be obtained for the different stages in the lifetime of a facility. The authorization process may differ among States but the stages in the lifetime of a nuclear fuel cycle facility: - Site evaluation; - Design; - Construction; - Commissioning; - Operation, including utilization and modification; - Decommissioning; - Release from regulatory control. For large or complex fuel cycle facilities, authorization by the regulatory body may be granted in several stages. Each stage may have a hold point and regulatory approval may be necessary to proceed to the next stage. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Governmental, Legal and Regulatory Framework for Safety | 0 |  |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 34 | 3.3 |
| Safety Standards Series No. SSG-12 | Licensing Process for Nuclear Installations | 0 |  |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the regulatory requirements for the content of the licensing documentation to be submitted by the operating organization in support of a licence application for fuel cycle facilities? [Topic 5.4] | A | F |
| Expectation | [NOTE: This primary question is only to reveal any specific information related to regulatory authorization as it applies to fuel cycle facilities. DO NOT repeat information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1) and GSR Part 4] The licensing documentation includes the safety analysis report and the operational limits and conditions or equivalent. The requirements specify a safety analysis report describes: - Demonstration of the safety of the facility; - All activities with safety significance in appropriate detail, including any restrictions on inputs to and outputs from the facility (e.g. limits on burnup and enrichment); - Explanation on the application of the safety principles and criteria in the design for the protection of operating personnel, the public and the environment; - Analysis of the hazards associated with the operation of the facility; - Demonstration of compliance with the regulatory requirements and criteria; - Safety analyses of accident sequences and of the safety features incorporated in the design for preventing accidents or minimizing the likelihood of their occurrence and for mitigating their consequences; - To the extent appropriate and in accordance with a graded approach, identification of the safety functions and the structures, systems and components (SSCs) important to safety; - The required intervals for periodic testing and inspection of SSCs important to safety. The licensing documentation shall be required to be maintained and updated during the operational lifetime of the facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The demonstration of safety of a fuel cycle facility is subject to a set of documents known as the licensing documentation (or safety case). The licensing documentation provides a basis for the safety of the facility at all stages of its lifetime and is updated periodically, to take account of modifications made to the facility and other changes. The licensing documentation shall be considered by the regulatory body in determining whether the authorizations necessary under national legislative and regulatory requirements are to be granted. The operational limits and conditions are the set of rules that establish parameter limits, the functional capability and the performance levels of equipment and personnel for the safe operation of a facility. The SSCs important to safety provide means for the prevention of the occurrence of postulated initiating events, the control and limitation of accident sequences, and mitigation of the consequences. Arrangements for maintenance, calibration, testing, surveillance and inspection demonstrate that items important to safety continue to: - Satisfy the intent of the design; - Ensure the continuing safety of the fuel cycle facility during maintenance; - Ensure the fuel cycle facility is returned to an authorized configuration on completion. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 4 (Rev. 1) | Safety Assessment for Facilities and Activities | 29 30 | Requirement 3 Requirement 4 |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 35 44 | Requirement 1 Requirement 5 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the provisions in place for the regulatory body to review and assess information to be submitted by operating organizations of nuclear fuel cycle facilities? [Topic 6.4] | A | F |
| Expectation | [NOTE: This primary question is only to reveal any specific information related to regulatory review and assessment as it applies to fuel cycle facilities. DO NOT repeat information that has already been provided in the core modules dealing with GSR Part 1 and GSR Part 4.] The regulatory body is required, and has the authority, to review and assess information provided by the operating organization in support of an application for authorization, as well as information provided by the operating organization that is required by any authorization. The findings of safety assessments and periodic safety reviews that have implications for safety are required to be reported to the regulatory body. The specificities of fuel cycle facilities are addressed in review and assessment process of the regulatory body. The nature of the nuclear fuel cycle facilities and the risks associated to it are considered according to a graded approach. The regulatory body takes account of associated chemical hazards and security advice when making its assessment. The evaluation of safety is based on criteria based on principles for safe design and operation, that are made available to the operating organizations. The regulatory body regularly reviews the effectiveness of the operating organization in managing operational safety of the facility, in implementing, applying and reviewing and revising as necessary operational safety programmes, and in ensuring continuing safety of the facility design. The application of the safety requirements for a nuclear fuel cycle is commensurate with the potential risk of the facility in accordance with a graded approach. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Review and assessment are undertaken to enable the regulatory body to make a decision or a series of decisions on the acceptability of the facility or activity in terms of safety. The process consists of examining the authorized party’s submissions, and other information on all aspects relating to the safety of the facility or activity. In the context of nuclear fuel cycle facilities, non-radiological hazards associated with the radioactive material in the facility need to be controlled, in order to minimize risks to workers, the public and the environment. Events initiated by chemical hazards and the toxic chemicals that are used in the facility can have a significant bearing on achieving the fundamental safety objective. Industrial hazards associated with radioactive material need to be considered in the design, commissioning and operation of the facility. Activities at nuclear fuel cycle facilities may include industrial processes that pose additional hazards to safe nuclear operations, site personnel and to the environment. Purely industrial hazards also need to be considered where these can interfere with the nuclear safety of the facility. The concept of defence in depth is applied to all safety related activities, whether organizational, behavioural or design related, in all operational states, and including activities with chemical hazards. For application of the safety requirements for any nuclear fuel cycle facility in accordance with a graded approach, the facility type and specific attributes (e.g., nature and the physical and chemical forms of the radioactive material, inventory of hazardous material, hazardous chemicals associated with the radioactive material; radioactive discharge and the storage of radioactive waste) is to be commensurate. with its potential hazards. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Governmental, Legal and Regulatory Framework for Safety | 49 | Requirement 25 Requirement 26 |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 36 44 58 | 3.10 Requirement 5 Requirement 11 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the provisions for regulatory inspections programme of fuel cycle facilities? [Topic 7.4] | A | F |
| Expectation | [NOTE: This primary question is only to reveal any specific information related to regulatory inspection as it applies to fuel cycle facilities. DO NOT repeat information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The regulatory inspection programme for fuel cycle facilities addresses: - Inspections of facilities and activities; - Types of inspections; - Application of the graded approach to inspection activities depending on the nature of the facilities and the risks associated to it, including chemical risks. Regulatory oversight activities deal with also the culture for safety of operating organizations. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | In the context of nuclear fuel cycle facilities, non-radiological hazards associated with the radioactive material in the facility need to be controlled, in order to minimize risks to workers, the public and the environment. Events initiated by chemical hazards and the toxic chemicals that are used in the facility can have a significant bearing on achieving the fundamental safety objective. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Inspections | 51 | Requirement 27, 4.53 |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 27 37 111 117 | 2.4. 3.13.; 3.14. 9.12. 9.41. |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| T | How leadership and culture for safety of authorized parties are overseen by the regulatory body in fuel cycle facilities? [Topic 7.4] | T | T |
| Expectation | The regulatory framework and the management system of the regulatory body specifies objective and arrangements, including guidance and procedures, for the oversight of leadership and culture for safety of the operating organization, notably through inspection, and review and assessment activities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Safety is the result of interaction of Human, Technology and Organization (HTO). Therefore, it is critical that regulatory bodies when exercising oversight on the safety of facilities and activities in conducting their regulatory functions, cover also human and organizational-related factors, including leadership for safety and culture for Safety of the organization. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Inspection | 52 | 4.53 |
| Safety Standards Series No. GSR Part 2 | Demonstration of leadership for safety by managers | 23 | Requirement 2 |
| Safety Standards Series No. GSR Part 2 | Fostering a culture for safety | 31 | Requirement 12 |
| Safety Standards Series No. GSR Part 2 | Measurement, assessment and improvement of leadership for safety and of safety culture | 33 | Requirement 14 |
| Safety Standards Series No. GSG-13 | Review and Assessment | 85 | 3.200 |
| Safety Standards Series No. GSG-13 | Inspection | 89 | 3.215; 3.220; 3.230 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements in place for operating organizations of fuel cycle facilities in relation to the management for safety, including to establish, implement, assess and continuously improve a management for ensuring that all safety requirements are met at all stages of the lifetime of the nuclear fuel cycle facility? [Topic 9.4] | A | F |
| Expectation | The legal and regulatory framework includes requirements addressing: - Responsibilities and accountabilities of staff; - Structure and the functions, roles and responsibilities of its personnel; - Lines of communication; - Sufficient qualified and experienced individuals; - Leadership and culture for safety; - Safety policy; - Resource management; - Management of safety-related processes and activities; - Establishment, assessment and continuously improvement of an integrated management system; - Reporting to the regulatory body;- interface arrangements; - Document management. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Safety has to be effectively manage including through the establishment and implementation of a comprehensive management system. The fuel cycle facility management comprises the members of the operating organization to whom the overall responsibility and the authority for the safety of the fuel cycle facility have been assigned. The safety policy established and implemented by the operating organization gives safety the utmost priority, overriding all other demands, including those of facility production and project schedules or research and development programmes. The safety policy promotes a strong culture for safety, including a questioning attitude and a commitment to excellent performance in all activities important to safety. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 38 39 40 110 | Requirement 2 Requirement 3 Requirement 4 Requirement 55 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in place in the regulatory framework for operating organizations to specify, monitor and manage the supply to it of items, products and services that may influence safety by? [Topic 9.4] | A | F |
| Expectation | Vendors, contractors and suppliers are required to either conform to the management system of the operating organization of the fuel cycle facility or implement their own management system that has been approved by the operating organization of the fuel cycle facility. In the case where the operating organization is not the design organization, the regulatory framework requires the design organization to establish and implement a management system for ensuring that all relevant safety requirements are met. The operating organization of the fuel cycle facility has processes and procedures in place for the effective supervision of all activities undertaken by vendors, contractors and suppliers. The operating organization of the fuel cycle facility has processes and procedures in place for the effective supervision of all activities undertaken by contractors. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that operating organizations of fuel cycle facilities maintain control over all activities and do not inadvertently delegate their responsibility for safety. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 2 | Management of the supply chain | 30 | Requirement 11 |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 42 | 4.16; 4.22; 8.4; 8.5; 8.22; 9.1; 9.6; 9.116 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for operating organizations of fuel cycle facilities to verify the safety of the facility? [Topic 9.4] | A | F |
| Expectation | The regulatory framework sets up requirements for: - Safety assessment; - Periodic safety review; - The establishment of an independent safety committee or an advisory group to advise the management of the operating organization on all safety aspects of the nuclear fuel cycle facility. The regulatory framework specifies that: - The adequacy of the design of the nuclear fuel cycle facility shall be verified by means of comprehensive safety assessment that results in a safety analysis report and that defines the operational limits and conditions required for safety - The safety of the facility or activity for all facility states shall be assessed in the safety analysis and shall be independently reviewed; - The operating organization shall conduct systematic safety assessments of the facility, in accordance with regulatory requirements, throughout the lifetime of the facility; - On the basis of the results of such periodic safety reviews, the operating organization shall implement any necessary corrective actions and shall consider the need for modifications to enhance safety. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that the safety of the fuel cycle facility is adequately reviewed at appropriate intervals to demonstrate that it continues to satisfy an adequate level of safety and will continue to do so for its remaining operational life. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 44 46 | Requirement 5 Requirement 6 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements within the regulatory framework relating to the main safety functions, and application of defence in depth at fuel cycle facilities? [Topic 9.4] | A | F |
| Expectation | The regulatory framework addresses measures to be taken, including for: - Application of defence in depth; - Confinement and cooling of radioactive material and associated harmful materials; - Protection against radiation exposure;- restricting the likelihood of events that might lead to a loss of control over a nuclear chain reaction, radioactive sources or any other source of radiation; - Mitigating the consequences of such events if they were to occur; - Criticality prevention; - Control of chemical hazards. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that the fundamental safety objective is met and that multiple levels of protection for all relevant safety activities - whether organizational, behavioural or equipment related - are incorporated as required in all aspects and at all stages of the life cycle of a fuel cycle facility. Application of the concept of defence in depth throughout the design and operation of a fuel cycle facility provides multilayer protection against a wide range of anticipated operational occurrences, including those resulting from equipment failure or human error within the facility, and from events that originate outside the facility. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 52 56 | Requirement 7 Requirement 10 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the design of a nuclear fuel cycle facility? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies requirements on the design of a nuclear fuel cycle facility to ensure the facility and items important to safety have the appropriate characteristics to ensure that the safety functions can be performed with the necessary reliability, that the facility can be operated safely within the operational limits and conditions for its entire lifetime and can be safely decommissioned, and that impacts on people, including to workers and other personnel at the facility, and the environment are as low as reasonably achievable. The regulatory framework specifies items important to safety to be identified and classified on the basis of their safety function and their safety significance, and to be designed in accordance with the relevant national and international codes and standards. |  |  |
| Help | Items important to safety are preferably of a design that has previously been proven in equivalent applications. This does not override the need for safety to be enhanced by the use of new or improved designs and technology, subject to appropriate qualification, testing and safety analysis. The design shall ensure radiation doses to workers and other personnel at the facility and to members of the public do not exceed the dose limits, and that doses are kept as low as reasonably achievable in operational states for the entire lifetime of the facility, and that they remain below acceptable limits and as low as reasonably achievable during, and following, accident conditions. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 53 54 59 61 | Requirement 8 Requirement 9 Requirement 12 Requirement 13 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements in relation to the design basis for items important to safety? [Topic 9.4] | A | F |
| Expectation | The regulatory framework requires a comprehensive safety analysis to be carried out in the design process, using methods of deterministic analysis, complemented by probabilistic assessments where appropriate, in accordance with a graded approach. The regulatory framework requires the determination of a set of design extension conditions deriving on the basis of deterministic analysis and engineering judgement with complementary probabilistic assessments, as appropriate. The regulatory framework specifies requirements on the design basis for items important to safety, including in relation to the capability, reliability and functionality for the relevant operational states, for accident conditions and for conditions arising from internal and external hazards, to meet the specific acceptance criteria over the lifetime of the facility. The regulatory framework specifies requirements on the identification of all foreseeable internal hazards and on the examination of all facility conditions that could directly or indirectly affect safety. The regulatory framework specifies requirements on the specification of design criteria corresponding to relevant physical parameters, for each operational state of the facility and for each design basis accident or equivalent. The regulatory framework specifies requirements on the application of engineering design rules to provide for safety margins such that no significant consequences would occur even if the operational limits were exceeded within these margins. The regulatory framework specifies requirements on the preparation of operational limits and conditions in the design stage, their confirmation in the commissioning stage and their establishment before operations of the facility commence. The regulatory framework specifies requirements on initiating events, including human induced events, that could affect safety and their effects, both individually and in credible combinations. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The design shall ensure, as far as is practicable, that accidents are not likely to occur over the operating lifetime of the nuclear fuel cycle facility. The design basis for each item important to safety is to be systematically justified and documented and the documentation shall provide all the necessary information for the operating organization to operate the nuclear fuel cycle facility safely and to maintain and, if necessary, eventually to replace the item with a replacement or substitute item meeting the design intent and all functional requirements of the original item. The comprehensive safety analysis, using methods of deterministic analysis and probabilistic assessments where appropriate aims to ensure that the design provides an adequate level of safety and meets the required design acceptance criteria. The determination of a set of design extension conditions aim to further improve the safety by enhancing its capabilities to withstand, without unacceptable consequences, accidents that are either more severe than design basis accidents or that involve additional failures. The design extension conditions is used to identify the additional accident scenarios to be addressed in the design and to plan practicable provisions for the prevention of such accidents or mitigation of their consequences. Initiating events, including human induced events, that could affect safety are to be identified and their effects, both individually and in credible combinations, are be evaluated. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 62 62 64 65 65 66 68 | Requirement 14 Requirement 15 Requirement 17 Requirement 18 Requirement 19 Requirement 20 Requirement 21 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for site evaluation? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies requirements on the evaluation of all foreseeable external events, both individually and in credible combinations. The requirements for site evaluation for fuel cycle facilities take account of SSR-1 requirements, including: - Site and regional characteristics; - Site specific hazards; - Natural and human induced external hazards; - Multiple nuclear installations on the same site or on adjacent sites; - Ultimate heat sink when required; - Seismic hazards; - Flooding hazards; - Extreme and rare meteorological events such as tornadoes and tropical cyclones; - Geotechnical and volcanic hazards; - External human induced hazards such as aircraft crashes and chemical explosions; - Monitoring of hazards throughout the lifetime of the fuel cycle facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that appropriate and comprehensive criteria are specified for the evaluation of the suitability of sites of fuel cycle facilities and that the results of the evaluation are fully documented. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 63 | Requirement 16 |
| Safety Standards Series No. SSR-1 |  | 27 |  |

SPECIFIC REQUIREMENTS FOR DESIGN

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements in relation to the resilience of the items important to safety, including buildings, whose failure due to internal or external events could compromise the main safety functions? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies that: - The design shall make adequate provision for redundancy, diversity and independence of equipment; - Provisions for radioactive waste management shall be considered at the design stage; - Design provisions shall be established for ensuring that discharges of gaseous, liquid and particulate radioactive material and associated hazardous chemicals to the environment comply with authorized limits; - Items important to safety shall be designed to facilitate maintenance, inspection and testing for their functional capability over the lifetime of the facility; - Human and organizational factors and the human–machine interface shall be considered throughout the design process; - The transfer of radioactive material and other hazardous material shall be considered in the safety analysis and the necessary controls shall be identified. The design shall provide features to ensure the safe transfer of radioactive material and associated chemicals. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | A single fault or equipment failure should not lead to accidents exceeding the conditions of the design basis. Adequate redundancy, diversity and independence with physical separation is provided for items important to safety to improve the reliability of functions important to safety. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 73 74 75 76 76 77 78 | Requirement 23 Requirement 24 Requirement 25 Requirement 26 Requirement 27 Requirement 28 Requirement 29 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements in relation to the lifetime of fuel cycle facilities? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - Items important to safety shall be designed to ensure the achievement of the design specifications and the required level of safety; - A qualification programme shall be implemented to verify that items important to safety are capable of performing their intended functions when necessary, and in the prevailing environmental conditions, throughout their design life; - The design shall include features as necessary to facilitate the commissioning process for the nuclear fuel cycle facility; - Design safety margins shall be adopted so as to accommodate the anticipated properties of items important to safety, to allow for the effects of materials ageing and degradation processes; - For the design consideration shall be given to facility the ultimate decommissioning of facilities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Items important to safety are designed so that they can be manufactured, constructed, assembled, installed and erected in accordance with established processes that ensure the achievement of the design specifications and the required level of safety. The qualification programme of items important to safety due take account conditions during maintenance and testing. Consideration in the design for decommissioning aims to keep the exposure of workers and the public, arising from decommissioning, as low as reasonably achievable and to ensure protection of people and protection of the environment, as well as to minimize the amount of radioactive waste generated in decommissioning. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 78 78 79 79 80 | Requirement 29 Requirement 30 Requirement 31 Requirement 32 Requirement 33 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements in relation to radiation protection? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The design shall ensure the protection of the public and the environment against uncontrolled releases of radioactive material in all facility states; - Releases shall be kept as low as reasonably achievable and within authorized limits in normal operation and within acceptable limits in accident conditions; - The design shall include means for the dynamic and static confinement of radioactive material and associated hazardous materials; - Leak detection shall be implemented as appropriate for the control of contamination; - Provision shall be made for ensuring that doses to workers and the public is kept as low as reasonably achievable, with account taken of the relevant dose constraints, and below the dose limits; - An adequate radiation monitoring in operational states, in design basis accidents and, if appropriate, in design extension conditions to be in place; - An adequate margin of subcriticality shall be ensure under operational states and conditions that are referred to as credible abnormal conditions, or conditions included in the design basis; - Cooling systems and the necessary support systems shall be provided to remove heat from radioactive decay and chemical reactions. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Provisions (Passive engineered control involving equipment design; active engineered control involving the use of process control instrumentation; chemical means; etc.) for ensuring subcriticality are first to prevent any criticality event, then, in relation to the installation of criticality detection and alarm systems, the identification and marking of appropriate evacuation routes and regrouping areas, the availability of appropriate emergency equipment, and specific measures to protect the public. Cooling systems aims to prevent overheating resulting in boiling or loss of confinement and the consequent release and dispersion of significant quantities of radioactive material from highly active systems for processing and storage of spent fuel, plutonium and other highly radioactive material. Alternate heat sinks have to be provided where the safety assessment identifies the potential for widespread consequences resulting from loss of the primary heat sink. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 81 82 83 84 85 91 | Requirement 34 Requirement 35 Requirement 36 Requirement 37 Requirement 38 Requirement 39 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for protection against non-radiological hazards? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The design shall include features to control reactive, flammable, corrosive and pyrophoric materials and mixtures used or produced in the processing of radioactive material; - Facilities shall be designed and located so as to prevent and control fires and to prevent explosions with potential radiological consequences and to minimize their effects; - The design shall ensure that personnel, the public and the environment are protected against toxic chemical exposures associated with radioactive material; - Instrumentation and control systems shall be provided for monitoring and control of all the process parameters that are necessary for safe operation in all operational states, for bringing systems to a safe state and for monitoring of accident conditions. - All instrumentation and control-based items important to safety shall be designed and arranged so that their safety functions can be adequately inspected and tested, and the systems important to safety can be maintained; - If a system is dependent upon computer-based equipment, appropriate standards and practices for the development and testing of computer hardware and software shall be established and implemented throughout the service life of the system, and in particular throughout the software development cycle. The entire software development cycle are to be subject to a quality management system; - Where control rooms and/or panels are needed for safety, including for emergency response, their accessibility and habitability shall be ensured by design to satisfy the requirements resulting from the safety assessment. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The chemistry of any reactive, pyrophoric, flammable or highly corrosive materials used or produced in the processing of nuclear materials are to be considered in the safety analysis. Examples of such materials are hydrogen, hydrofluoric acid and red oil, which may be used or produced in processes including dissolution, extraction and fuel fabrication. The reliability, redundancy and diversity required of instrumentation and control systems are to be proportionate to their safety classification. The entire software development cycle are to be subject to a quality management system. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 91 92 93 94 96 96 97 | Requirement 40 Requirement 41 Requirement 42 Requirement 43 Requirement 44 Requirement 45 Requirement 46 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for emergency systems? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The design of a nuclear fuel cycle facility shall include adequate provisions to enable prompt response to an emergency; - The need for an emergency response facility, on or near the site, shall be determined; - The design of electrical power supply systems shall ensure their required availability, sustainability and reliability, with provisions for an emergency power supply where necessary. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Provisions to enable prompt response to an emergency include alarms, escape routes and means for monitoring, communication and accounting for personnel. The safety assessment to determine the need for an emergency response facility, on or near the site, provide the basis for the coordination on-site response if necessary |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 97 98 98 | Requirement 47 Requirement 48 Requirement 49 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for compressed air systems, handling and storage of fissile material and other radioactive material, and monitoring and analysis of process chemistry? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The provisions for compressed air systems relied upon for safety functions; - Handling and storage of fissile material and other radioactive material. - Monitoring and analysis of process chemistry. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Provisions to enable prompt response to an emergency include alarms, escape routes and means for monitoring, communication and accounting for personnel. The safety assessment to determine the need for an emergency response facility, on or near the site, provide the basis for the coordination on-site response if necessary |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 99 99 101 | Requirement 50 Requirement 51 Requirement 52 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for construction of a nuclear fuel cycle facility? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - Items important to safety to be constructed, assembled, installed and erected in accordance with established processes that ensure that the design specifications and design intent are me; - The safety implications of design changes during construction shall be assessed and documented; - Records shall be maintained to demonstrate the facility and its equipment have been constructed in accordance with the design specifications; - Following construction of the facility, the operating organization shall review the as built documents to confirm that the design intent has been met and the safety functions specified will be fulfilled. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The construction of a nuclear fuel cycle facility shall start only after the operating organization has verified that the main safety issues in the design have been resolved and has demonstrated conformity of the design with the relevant regulatory requirements. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 102 | Requirement 53 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the commissioning of a nuclear fuel cycle facility? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies the operating organization shall ensure that a commissioning programme for the nuclear fuel cycle facility is established and implemented. The results and analyses of tests directly affecting safety are made available to the regulatory body for review and approval as appropriate. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | An adequate commissioning programme is prepared for the testing of the components and systems of the nuclear fuel cycle facility after their construction or modification to demonstrate that they meet the design objective and the performance criteria. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 103 | Requirement 54 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the personnel of fuel cycle facilities? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies that: - Fuel cycle facility shall be staffed with competent managers and sufficient qualified personnel for the safe operation of the facility; - All activities that may affect safety shall be performed by suitably qualified and competent persons. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 111 116 | Requirement 56 Requirement 58 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the operation of fuel cycle facilities? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The nuclear fuel cycle facility is operated in accordance with the set of operational limits and conditions; - All safety related activities shall be adequately analysed and controlled to ensure that the risks associated with ionizing radiation and associated toxic chemicals are kept as low as reasonably achievable; - An effective ageing management programme shall be implemented to manage the ageing of items important to safety so that the required safety functions are fulfilled over the entire operating lifetime of the nuclear fuel cycle facility; - The operating organization shall establish and implement a programme for the control of modifications to the facility; - The operating organization shall establish and maintain a system for the control of records and reports in the nuclear fuel cycle facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Facilities are operated within a comprehensive set of operational limits and conditions to prevent situations arising that could lead to anticipated operational occurrences or accident conditions, and to mitigate the consequences of such events if they do occur. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 114 118 119 120 121 | Requirement 57 Requirement 59 Requirement 60 Requirement 61 Requirement 62 |

FACILITY OPERATIONS

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the operating procedures, and operational housekeeping and material conditions? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - Operating procedures shall be developed that apply comprehensively for normal operation, anticipated operational occurrences and accident conditions, in accordance with the policy of the operating organization and regulatory requirements; - The operating organization shall develop and implement programmes to maintain a high standard of material conditions, housekeeping and cleanliness in all working areas. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 123 123 | Requirement 63 Requirement 64 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the maintenance, periodic testing and inspection? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies the operating organization shall ensure that effective programmes for maintenance, periodic testing and inspection are established and implemented. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Maintenance, periodic testing and inspection are conducted to ensure that items important to safety are able to function in accordance with their design intent and safety requirements, in compliance with the operational limits and conditions, and shall support the long-term safety of the facility. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 124 | Requirement 65 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for all operations with fissile material to be carried out, to maintain an adequate margin of subcriticality, under operational states and conditions that are referred to as credible abnormal conditions or conditions included in the design basis? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies All operations with fissile material shall be carried out to maintain an adequate margin of subcriticality, under operational states and conditions that are referred to as credible abnormal conditions or conditions included in the design basis. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The criticality safety programme ensures operators are aware of the criticality hazard. Operators are trained in and made aware of the conditions that may cause a criticality. All operations to which nuclear criticality safety is pertinent is governed by approved procedures. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 125 | Requirement 66 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the radiation protection programme and management of radioactive waste and effluents? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The operating organization shall establish and implement a radiation protection programme; - The operating organization shall establish and implement a programme for the management of radioactive waste and effluents. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The radiation protection programme ensure that for all operational states and design basis accidents, doses due to exposure to ionizing radiation and doses due to any discharges of radioactive material from the facility are kept as low as reasonably achievable and are below authorized limits The objective is to ensure the facility is operated so as to control and minimize, as far as reasonably achievable, the generation of radioactive waste of all kinds in terms of both activity and volume. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 128 130 | Requirement 67 Requirement 68 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the operational safety programmes? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies: - The operating organization shall make arrangements for ensuring protection against fire and explosion; - The operating organization shall establish and implement a programme for controlling the risks associated with industrial and chemical hazards to workers and the public and shall keep the risks as low as reasonably achievable; - The operating organization shall establish an accident management programme based on the results of the safety analysis; - The operating organization shall establish arrangements for on-site preparedness for, and response to, a nuclear or radiological emergency; - The operating organization shall establish a programme to learn from events at the facility and events at other nuclear fuel cycle facilities and in the nuclear industry worldwide; Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The arrangements for ensuring fire safety cover: - Adequate management for fire safety; - Preventing fires from starting; - Detecting and quickly extinguishing any fires that do start; - Preventing the spread of those fires that have not been extinguished; and - Providing sufficient protection from fire to permit the facility to be brought to a safe and stable state. The industrial and chemical safety programme includes arrangements for the planning, implementation, monitoring and review of the relevant preventive and protective measures. It has to be compatible with the requirements for nuclear and radiation safety. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 132 133 134 134 137 | Requirement 69 Requirement 70 Requirement 71 Requirement 72 Requirement 73 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for the preparation for decommissioning? [Topic 9.4] | A | F |
| Expectation | The regulatory framework specifies that the operating organization shall prepare a decommissioning plan and shall maintain it throughout the lifetime of the facility, unless otherwise approved by the regulatory body, to show that decommissioning can be accomplished safely and in such a way as to meet the defined end state. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | For a new facility, planning for decommissioning begins in the design stage. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-4 | Safety of Nuclear Fuel Cycle Facilities | 138 | Requirement 74 |