| Questionnaire Title | Facilities and Activities and Exposure Situations |
| --- | --- |
| Module Name | Regulation of Research Reactors |

| QTYPE | 1.Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the provisions in place for the authorization (licensing) of research reactors during the different phases of their life cycle? [Topic 5.3] | A | F |
| Expectation | [Note: This primary question is only to reveal any specific information related to regulatory authorization as it applies to research reactors. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The provisions for the licensing of research reactors during the different phases in their life cycle include: - site evaluation; - design; - construction; - commissioning; - operation, including utilization and modification; - decommissioning; - release from regulatory control. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The general requirements for regulatory authorization are covered by the core question sets dealing with GSR Part 1 (Rev.1). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Governmental, Legal and Regulatory Framework for Safety | 0 |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 34 | 3.4-3.5 |

| QTYPE | 2. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the regulatory requirements for the content of the safety analysis report to be submitted by the operating organization in support of a licence application for a research reactor? [Topic 5.3] | A | F |
| Expectation | [Note: This primary question is only to reveal any specific information related to content of the safety analyses report as it applies to research reactors. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The requirements address the content of the safety analysis report including: -detailed description of the reactor site, reactor, and all facilities and activities with safety significance; - general safety principles and criteria to be applied to the design; - analysis of potential hazards associated with operation of the reactor; - safety analyses of accident sequences; - safety features to avoid or minimize likelihood of accidents or mitigate consequences in accordance with the defence in depth concept; - information for establishing the operational limits and conditions (OLCs) for the reactor; conduct of operations; - management system programme for all stages of reactor life and details of the emergency plan of the research reactor. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The safety analysis report includes information to demonstrate compliance with national legislation and requirements issued by the regulatory body. The level of detail of the information to be presented in the safety analysis report is to be determined using a graded approach. For reactors with high power levels, the safety analysis report usually requires more detail in discussions such as those of reactor design and accident scenarios. For some reactors (e.g. research reactors with low potential hazard, critical or subcritical assemblies) the requirements for the safety analysis report content may be much less extensive. Please also refer to safety guide SSG-20 (Safety Assessment for Research Reactors and Preparation of the Safety Analysis Report). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 34 | Requirement 1 |
| Safety Standards Series No. guide SSG-20 | Safety Assessment for Research Reactors and Preparation of the Safety Analysis Report | 55 |  |

| QTYPE | 3. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What provisions are in place for the regulatory body to review and assess the information to be submitted by research reactor operating organization? [Topic 6.3] |  |  |
| Expectation | [Note: This primary question is only to reveal any specific information related to regulatory review and assessment process as it applies to research reactors. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The regulatory body has the authority and responsible to review and assess information provided by the operating organization in support of an application for authorization, as well as information provided by the operating organization that is required by any authorization. The regulatory body regularly reviews the effectiveness of the operating organization in managing operational safety of the research reactors, in implementing, applying and reviewing and revising as necessary operational safety programmes, and in ensuring continuing safety of the plant design. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Please also refer to safety guides SSG-20 (Safety Assessment for Research Reactors and Preparation of the Safety Analysis Report) and GSG-13 (Functions and Processes of the Regulatory Body for Safety). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Governmental, Legal and Regulatory Framework for Safety | 0 |  |
| Safety Standards Series No. GSR Part 4 (Rev. 1) | Safety Assessment for Facilities and Activities | 0 |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 36 | 3.11-3.12 |

| QTYPE | 4. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the provisions in place for a regulatory inspection programme for research reactors? [Topic 7.3] |  |  |
| Expectation | [Note: This primary question is only to reveal any specific information related to regulatory inspection as it applies to research reactors. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The regulatory inspection programme for research reactors include inspections of facilities and activities, including experiment facilities and activities. The graded approach is applied in the development of the inspection programme. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to verify how the regulatory inspection activities are planned, performed and analysed for research reactors. Please also refer to safety guides SSG-24 (Safety in the Utilization and Modification of Research Reactors) and GSG-13 (Functions and Processes of the Regulatory Body for Safety). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Governmental, Legal and Regulatory Framework for Safety | 51 |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 36 | 3.13 |

| QTYPE | 5. Primary Question | SA | IL |
| --- | --- | --- | --- |
| T | How are leadership and culture for safety of authorized parties overseen by the regulatory body in research reactors? [Topic 7.3] | T | T |
| Expectation | The regulatory framework and the management system of the regulatory body specifies objective and arrangements, including guidance and procedures, for the oversight of leadership and culture for safety of the operating organization, notably through inspection, and review and assessment activities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Safety is the result of interaction of Human, Technology and Organization (HTO). Therefore, it is critical that when conducting their functions, regulatory bodies exercise oversight on the safety of facilities and activities also covering human and organizational-related factors, including leadership for safety and culture for Safety of the organization. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) | Inspection | 52 | 4.53 |
| Safety Standards Series No. GSR Part 2 | Demonstration of leadership for safety by managers | 23 | Requirement 2 |
| Safety Standards Series No. GSR Part 2 | Fostering a culture for safety | 31 | Requirement 12 |
| Safety Standards Series No. GSR Part 2 | Measurement, assessment and improvement of leadership for safety and of safety culture | 33 | Requirement14 |
| Safety Standards Series No. GSG-13 | Review and Assessment | 85 | 3.200 |
| Safety Standards Series No. GSG-13 | Inspection | 89 | 3.215; 3.220; 3.230 |

| QTYPE | 6. Primary Question |  | SA | IL |
| --- | --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements relating to the fundamental safety objective, measures to achieve the fundamental safety objective, fundamental safety functions and application of defence in depth concept at research reactors? [Topic 9.3] |  |  |  |
| Expectation | The regulatory framework addresses measures to be taken, including: - control of radiation exposure to people and the release of radioactive material to the environment; - restricting the likelihood of events that might lead to a loss of control over a nuclear reactor core, nuclear chain reaction, radioactive sources or any other source of radiation; - mitigating the consequences of such events if they were to occur; control of reactivity; - removal of heat from the reactor and from the fuel store; - confinement of radioactive material and application of defence in depth. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |  |
| Help | The objective is to ensure that multiple levels of protection for all relevant safety activities, whether organizational, behavioural or equipment related are incorporated as required in all aspects and at all stages of the life cycle of a research reactor. Application of the concept of defence in depth throughout the design and operation of a research reactor provides multilayer protection against a wide range of anticipated operational occurrences, including those resulting from equipment failure or human error within the facility, and from events that originate outside the facility. |  |  |  |
| Question Tags (one row per Tag) |  |  |  |  |
| Category | Name |  |  |  |
| References (one row per reference) |  |  |  |  |
| Name | Comments | Page | Para |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 48 | 6.1-6.5; Requirement 9, 10 |  |

| QTYPE | 7. Primary Question | SA |  | IL |
| --- | --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for operating organizations of research reactors to monitor and control all activities performed by vendors, contractors and suppliers? [Topic 9.3] |  |  |  |
| Expectation | Vendors, contractors and suppliers are required to either conform to the management system of the operating organization of the research reactor or implement their own management system that has been approved by the operating organization of the research reactor. In the case where the operating organization is not the design organization, the regulatory framework requires the design organization to establish and implement a management system for ensuring that all relevant safety requirements are met. The operating organization has processes and procedures in place for the effective supervision of all activities undertaken by vendors, contractors and suppliers. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |  |
| Help | The prime responsibility for safety of the operating organization covers all the safety-related activities regarding the operation directly and indirectly, including activities for experiments. The operating organization for a research reactor is responsible for maintaining control over all activities and does not inadvertently delegate their responsibility for safety. Please also refer to safety guides SSG-84 (The Operating Organization and the Recruitment, Training and Qualification of Personnel for Research Reactors), and GS-G-3.1 (Application of the Management System for Facilities and Activities). |  |  |  |
| Question Tags (one row per Tag) |  |  |  |  |
| Category | Name |  |  |  |
| References (one row per reference) |  |  |  |  |
| Name | Comments | Page | Para |  |
| Safety Standards Series No. GSR Part 2 | Leadership and management for safety | 30 | Requirement 11 |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 38 97 | Requirement 2, para 4.3; Requirement 67 |  |

| QTYPE | 8. Primary Question |  | SA | IL |  |
| --- | --- | --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operating organizations of research reactors to establish, implement, assess and continually improve a management system that integrates safety, health and environmental elements to ensure safe operation? [Topic 9.3] |  |  |  |  |
| Expectation | The regulatory framework includes requirements addressing: - safety policies that give safety the highest priority; - responsibility in the management for safety and accountabilities of staff; - financial resources for safety; - supervision of activities delegated by the operating organization to other organizations; safety, health and environmental policies; - lines of communication; - maintaining sufficient staff with appropriate qualifications and trainings at all levels; - development of management system processes; - safety culture; - reporting and submission to the regulatory body; - structure and responsibilities of the operating organization; - interface arrangements; - document management; - review, monitoring and audit all safety related matters on a regular basis, with appropriate corrective actions where necessary. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |  |  |
| Help | The operating organization has to establish a comprehensive management system in compliance with the GSR Part 2. The research reactor management comprises the members of the operating organization to whom the overall responsibility and the authority for the safety of the research reactor facility have been assigned. Please also refer to safety guides SSG-84 (The Operating Organization and the Recruitment, Training and Qualification of Personnel for Research Reactors), and GS-G-3.1 (Application of the Management System for Facilities and Activities). |  |  |  |  |
| Question Tags (one row per Tag) |  |  |  |  |  |
| Category | Name |  |  |  |  |
| References (one row per reference) |  |  |  |  |  |
| Name | Comments | Page |  |  | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors |  | 39 |  | Requirement 4, 4.7-4.20 |
| Safety Standards Series No. GSR Part 2 | The Management system |  | 26 |  |  |

| QTYPE | 9. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operating organizations of research reactors to verify the adequacy of the design of the research reactor? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies the adequacy of the design of the research reactor shall be verified by means of comprehensive deterministic safety analysis and complementary probabilistic analysis as appropriate and shall be validated by independent verification by individuals or groups independent from those who originally performed the design work. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that the safety of the research reactor is adequately reviewed at appropriate intervals to demonstrate that it continues to satisfy an adequate level of safety and will continue to do so for its remaining operational life. Please also refer to safety SSG-20 (Safety Assessment for Research Reactors and Preparation of the Safety Analysis Report). The safety assessment is continued throughout all the stages of the reactor’s lifetime and is conducted in accordance with the potential magnitude and nature of the hazards associated with the particular facility or activity. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 43 | Requirement 5 |
| Safety Standards Series No. GSR Part 4 (Rev. 1) | Safety Assessment for Facilities and Activities | 0 |  |

| QTYPE | 10. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements relating to radiation protection in design for research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies that the design of a research reactor shall ensure that radiation doses to workers at the facility and to members of the public: - does not exceed the dose limits; - are kept as low as reasonably achievable in operational states for the entire lifetime of the facility; - remain below acceptable limits and as low as reasonably achievable in, and following, accident conditions. The regulatory framework includes requirements on design for radiation protection, design of systems containing fissile or radioactive material, to radiation monitoring and to treatment of waste and effluents in order to keep discharges below authorized limits and as low as reasonable achievable. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that the design of the research reactor results in exposure of workers and the public to radiation being within regulatory limits and kept as low as reasonably achievable (ALARA). Please also refer to safety guides SSG-85 (Radiation Protection and Radioactive Waste Management in the Design and Operation of Research Reactors) and SSG-27 (Criticality Safety in the Handling of Fissile Material). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 | Radiation Protection and Safety of Radiation Sources: International Basic Safety Standards | 0 |  |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 50 | Requirement 8; para 6.8 |

| QTYPE | 11. Primary Question | SA | IL |
| --- | --- | --- | --- |
|  | Within the regulatory framework, what are the requirements relating to implementation of safety measures, security measures and arrangements for the State system of accounting for, and control of, nuclear material for research reactors in and integrated manner to the design? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies that safety measures, nuclear security measures and arrangements for the State system of accounting for, and control of, nuclear material for research reactors shall be designed and implemented in an integrated manner so that they do not compromise one another. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The purpose of this question is to determine if consideration is included in the regulatory framework for the integration of safety with nuclear security and the State system for accounting for, and control of, nuclear material. In particular, that safety measures are not inadvertently defeated by either of the other two. It is not to determine whether the nuclear security or State system for accounting and control are adequate. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 53 | Requirement 11 |

| QTYPE | 12. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements relating to design features to facilitate radioactive waste management and decommissioning of research reactors? [Topic 9.3] |  |  |
| Expectation | In addressing the design provisions for the management of radioactive waste, the following shall be considered: - avoidance and optimisation of the generation of radioactive waste, including from experiment facilities; - management of generated waste; predisposal and disposal routes for waste; - compliance with authorized discharge limits; - doses to the public and effects on the environment are as low as reasonably achievable; monitoring of discharges. In addressing the design provisions for decommissioning, the following shall be considered: - keeping the exposure of workers, the public and environment from decommissioning activities to as low as reasonably achievable; - minimizing the number and size of contaminated areas; choosing materials that can be stored in the facility, that are resistant to all chemicals in use and that have sufficient wear resistance; - avoidance of undesired accumulations of chemical or radioactive materials; - remote decontamination where necessary; - amenability to treatment, interim storage, transport and disposal of waste; - keeping the design documentation and records available throughout the lifetime of the facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure the design of the facility optimizes generation of waste during the life of the facility and that all regulatory requirements for discharges and disposal are satisfied. Please also refer to safety guides SSG-85 (Radiation Protection and Radioactive Waste Management in the Design and Operation of Research Reactors) and SSG-47 (Decommissioning of Nuclear Power Plants and Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 55 | Requirement 15; 6.27-6.28 |

| QTYPE | 13. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the general requirements for the design of research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework covers general and specific design requirements for research reactors as defined in SSR-3 (From Req. 16 to 34 and from Req. 36 to 41), including on: - Safety analysis of the design, including using DSA and PSA; - Safety classification of structures, systems and components; - Design basis for items important to safety; - Postulated initiating events; - Internal and external hazards; - Design basis accidents, design limits and design extension conditions; - Engineered safety features; - Reliability of items important to safety; - Single failure criterion, common cause failures, fail-safe design; - Physical separation and independence of safety systems; - Qualification, calibration, testing, maintenance, repair, replacement, inspection and monitoring of items important to safety; - Design for commissioning, ageing management and decommissioning; - Design for emergency preparedness and response; - Design for radiation protection; - Provision for safe utilization and modification; and for long shutdown periods - Prevention of unauthorized access to, or interference with, items important to safety, and of disruptive or adverse interactions between systems important to safety. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | In view of the large variety in designs and utilization programmes of research reactors worldwide, it is expected that the requirements cover only those which are operating, under construction or planned in the Member State. E.g., a country with only one research reactor that is not capable of generating a large fission product inventory or decay heat load may not have regulatory requirements for off-site emergency planning. Please also refer to safety guides SSG-37 (Instrumentation and Control Systems and Software Important to Safety for Research Reactors) and SSG-81 (Maintenance, Periodic Testing and Inspection of Research Reactors), SSG-85 (Radiation Protection and Radioactive Waste Management in the Design and Operation of Research Reactors) and NS-G-4.3 (Core Management and Fuel Handling for Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 55 | Requirement 16 to 34; Requirement 36 to 41 |

| QTYPE | 14. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are the requirements for identification and evaluation of all foreseeable internal hazards and external hazards for a research reactor, including the potential for human induced events directly or indirectly to affect the safety of the research reactor. [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies, that all foreseeable internal hazards and external hazards for a research reactor, including the potential for human induced events directly or indirectly to affect the safety of the research reactor, are identified and their effects, both individually and in credible combinations, are evaluated. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure appropriate and comprehensive requirements are specified for consideration of hazards in designing the layout of the facility and in determining the postulated initiating events and generated loadings for use in the design of relevant items important to safety for the reactor facility. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 58 | Requirement 19 |

| QTYPE | 15. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements relating to optimal operator performance and human factors for the design and safe operation of research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies that a systematic consideration of human factors, including the human–machine interface, is applied at an early stage in the design process for a research reactor facility, including its experimental facilities, and shall be continued throughout the entire design process. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Please also refer to safety guide SSG-84 (The Operating Organization and the Recruitment, Training and Qualification of Personnel for Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Design for optimal operator performance | 72 | Requirement 35, 6.103-6.107 |

| QTYPE | 16. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operating organizations of research reactors to develop and maintain the competence of personnel? [Topic 9.3] |  |  |
| Expectation | The regulatory framework includes requirements for operating organizations to establish minimum qualifications for personnel and to develop and deliver training programmes that: are commensurate with the assigned functional responsibilities and authorities; use a systematic approach to training; include update and refresher training; cover all operational states of the research reactor; address research reactor modifications in a timely manner. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that only suitably qualified and experienced personnel perform duties which can affect safety. Programmes should be established for senior managers, shift supervisors, operators, radiation protection personnel, maintenance personnel, quality assurance personnel and others working at the research reactor facility whose activities can affect safety. Please also refer to safety guide SSG-85(The Operating Organization and the Recruitment, Training and Qualification of Personnel for Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 101 | Requirement 69, 7.13-7.27 |

| QTYPE | 17. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operational limits and conditions (OLCs) of research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework includes requirements which address: - overall objectives of OLCs; - safety limits; - safety system set points; - limiting conditions for safe operation; - inspection, periodic testing and maintenance; - administrative requirements; and - violation of OLCs. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The OLCs should be used to provide the framework for the safe operation of the research reactor. OLCs should be prepared for each stage in the lifetime of the reactor (e.g. commissioning and operation). Please also refer to safety guide SSG-83 (Operational Limits and Conditions and Operating Procedures for Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 104 | Requirement 71, 7.32-7.43 |

| QTYPE | 18. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for the performance of safety related activities? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies requirements: - The operating organization for a research reactor facility shall ensure that safety related activities are adequately analysed and controlled to ensure that the risks associated with harmful effects of ionizing radiation are kept as low as reasonably achievable; - No experiments can be conducted without adequate review and justification. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | All routine and non-routine operational activities are assessed for potential risks associated with harmful effects of ionizing radiation. All activities important to safety are carried out in accordance with approved written procedures to ensure that the research reactor is operated within the established operational limits and conditions. Research reactors, by their nature, will frequently include experimental devices and experiment staff which can generate challenges to safety that can vary substantially from one experimental campaign to another one. Please also refer to safety guides SSG-83 (Operational Limits and Conditions and Operating Procedures for Research Reactors), NS-G-4.3 (Core Management and Fuel Handling for Research Reactors), SSG-12 (Licensing Process for Nuclear Installations), and SSG-24 (Safety in the Utilization and Modification of Research Reactors). |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 107 | Requirement 72 |

| QTYPE | 19. Primary Question | SA | IL |
| --- | --- | --- | --- |
|  | Within the regulatory framework, what are requirements for operating organizations of research reactors to establish and implement a commissioning programme? [Topic 9.3] |  |  |
| Expectation | If there is an on-going commissioning process or such a process is expected in the foreseeable future for a research reactor, the regulatory framework specifies that the operating organization shall ensure that a commissioning programme for the research reactor is established and implemented. The requirements may also apply to the restart of a research reactor after a lengthy shutdown period or the commissioning of experiment facilities and major facility modifications. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that all SSCs important to safety are systematically commissioned and demonstrate they provide the required safety functions to the performance standards specified in the design. The importance to safety of a comprehensive well executed commissioning programme for a facility cannot be overstated. Commissioning tests often uncover deficiencies with SSCs that might otherwise not have been discovered until an SSC is required to respond to a plant upset or accident situation. Please also refer to safety guides SSG-12 (Licensing Process for Nuclear Installations) and SSG-80 (Commissioning of Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 107 | Requirement 73; 7.47-7.56 |

| QTYPE | 20. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operating organizations of research reactors to develop operating instructions and procedures before operation commences? [Topic 9.3] |  |  |
| Expectation | The regulatory framework includes requirements addressing: - preparation of operational limits and conditions; - development of operating instructions; - operator compliance with operating instructions; - review and update of operating instructions and - identification and investigation of deviations from operating instructions. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that operations which may affect safety in operational states and in accident conditions are conducted according to written instructions. Please also refer to safety guide SSG-83 (Operational Limits and Conditions and Operating Procedures for Research Reactors) |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 110 | Requirement 74; 7.57-7.62 |

| QTYPE | 21. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements for operating organizations of research reactors to conduct maintenance, calibration, testing, surveillance and inspection to ensure that SSCs important to safety are able to function in accordance with the design intent and with safety requirements? [Topic 9.3] |  |  |
| Expectation | The regulatory framework requires that maintenance, calibration, testing, surveillance and inspection to be performed using written procedures and work permits at frequencies defined by the safety case and using equipment and items that are identified and controlled. The regulatory framework requires records of maintenance, calibration, testing, surveillance and inspection to be kept and assessed. The regulatory framework requires the maintenance, calibration, testing, surveillance and inspection programme to be reviewed. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that arrangements for maintenance, calibration, testing, surveillance and inspection demonstrate that SCCs important to safety continue to satisfy the intent of the design, ensure the continuing safety of the research reactor during maintenance and ensure the research reactor is returned to an authorized configuration on completion. Special attention should be given to subordinate operations such as decontamination, washing and preparation for maintenance. For some research reactors of high potential hazard, additional safety measures may be needed, such as non-permanent equipment to be used for accidents more severe than the design basis. Please also refer to safety guides SSG-83(Operational Limits and Conditions and Operating Procedures for Research Reactors) and SSG-81(Maintenance, Periodic Testing and Inspection of Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 112 | Requirement 77, 7.68-7.76 |

| QTYPE | 22. Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Within the regulatory framework, what are requirements pertaining to the operational safety of research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework includes requirements addressing: - core management and fuel handling; - fire safety; - records and reports; - utilization and modification of the reactor; - safety assessments during the operational lifetime, taking into account ageing related aspects and extended shutdowns; and - feedback of operating experience. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Please refer to safety guides SSG-83 (Operational Limits and Conditions and Operating Procedures for Research Reactors), NS-G-4.3 (Core Management and Fuel Handling for Research Reactors), SSG-10 (Ageing Management for Research Reactors), SSG-81 (Maintenance, Periodic Testing and Inspection of Research Reactors) and SSG-24 (Safety in the Utilization and Modification of Research Reactors). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 114 | Requirement 78, 82, 83, 88; 7.77-7.87; 7.94-7.106; 7.123-7.129 |

| QTYPE | 23. Primary Question | SA | IL |
| --- | --- | --- | --- |
|  | Within the regulatory framework, what are requirements for operating organizations of research reactors to establish and implement provisions for emergency preparedness and response? [Topic 9.3] |  |  |
| Expectation | The regulatory framework specifies that the operating organization shall prepare emergency plans that address: - Maintaining protection and safety; - Mitigating consequences of accidents; - Protection of site personnel and the public; - Coordination of activities with other response organizations; - Timely communication with the public; emergency organizational structure; - Testing and validation of plans; - Review and updating of plans; - Training programme; facilities, instruments, tools, equipment, documentation and communication systems; - Accident management programme and -Provision of relevant information on safety parameters. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Emergency procedures should be based on the accidents analysed in the safety analysis report as well as those additionally postulated for the purposes of emergency planning. Please also refer to safety guides SSG-85 (Radiation Protection and Radioactive Waste Management in the Design and Operation of Research Reactors), GSG-2 (Criteria for Use in Preparedness and Response for a Nuclear or Radiological Emergency) and GS-G-2.1 (Arrangements for Preparedness for a Nuclear or Radiological Emergency). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 117 | Requirement 81; Para 7.89-7.93 |
| Safety Standards Series No. GSR Part 7 | Preparedness and Response for a Nuclear or Radiological Emergency | 0 |  |

| QTYPE | 24. Primary Question | SA | IL |
| --- | --- | --- | --- |
|  | Within the regulatory framework, what are requirements for operating organizations of research reactors to establish and implement a radiation protection programme? [Topic 9.3] |  |  |
| Expectation | The regulatory framework addresses the specified requirements for radiation protection programme, radiation protection personnel, reference levels, control of occupational exposures and contamination control. All activities involving potential radiation exposure are planned, supervised and monitored. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that adequate arrangements are implemented to protect workers against the effects of ionizing radiation. Please also refer to safety guides RS-G-1.8 (Environmental and Source Monitoring for Purposes of Radiation Protection), GSG-7 (Occupational Radiation Protection), SSG-44 (Establishing the infrastructure for radiation safety), GS-G-1.5 (Regulatory Control of Radiation Sources) and GSG-9 (Regulatory Control of Radioactive Discharges to the Environment). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-3 | Safety of Research Reactors | 121 | Requirement 84 |
| Safety Standards Series No. GSR Part 3 | Radiation Protection and Safety of Radiation Sources: International Basic Safety Standards | 0 |  |

| QTYPE | 25. Primary Question | SA | IL |
| --- | --- | --- | --- |
|  | Within the regulatory framework, what are requirements relating to the preparation for decommissioning of research reactors? [Topic 9.3] |  |  |
| Expectation | The regulatory framework includes requirements addressing: - preparation and maintenance of a decommissioning plan; - funding arrangements; approval by regulatory body; - storage, treatment, transport and disposal of waste; - retaining the appropriate resources expertise and knowledge; - records and documentation and - review and updating of the plan. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The objective is to ensure that adequate arrangements are in place for the safe decommissioning of research reactors. The plan should show that decommissioning can be accomplished safely to meet the defined end state. Please also refer to safety guides SSG-12 (Licensing Process for Nuclear Installations), WS-G-5.2 (Safety Assessment for the Decommissioning of Facilities Using Radioactive Material) and SSG-47(Decommissioning of Nuclear Power Plants, Research Reactors and other Nuclear Fuel Cycle Facilities) |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No.SSR-3 | Safety of Research Reactors | 126 | Requirement 89 |
| Safety Standards Series No. GSR Part 6 | Decommissioning of Facilities | 0 |  |