| Questionnaire Title | Facilities and Activities and Exposure Situations |
| --- | --- |
| Module Name | Safety Requirements for Disposal of Radioactive Waste |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements for authorization of a radioactive waste disposal facility? [Topic 5.5] | B | F |
| Expectation | [Note: The part of this primary question related to Authorization is only to reveal any specific information related to authorization as it applies to waste disposal facilities and activities. DO NOT REPEAT any information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] There are regulatory requirements for registrants and licensees to notify the regulatory body of any intention to introduce a disposal facility. Authorization by the regulatory body is a pre-requisite for all disposal facilities that are not either explicitly exempted or approved by means of a notification process. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 61 | Requirement 7 |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 46 | Requirement 23 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the review and assessment of radioactive waste disposal facilities and activities, that the regulatory body performs for determining compliance with regulatory requirements. [Topic 6.5] | A | F |
| Expectation | [Note: This primary question is only to reveal any specific information related to regulatory review and assessment as it applies to disposal facilities and activities. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The regulatory body reviews and assesses relevant information to determine whether radioactive waste disposal facilities and activities comply with regulatory requirements. The regulatory body has established a regulatory system for protection and safety that includes review and assessment of radioactive waste disposal facilities and activities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 49 | Requirement 25 and 26 |
| Safety Standards Series No. GSR Part 3 |  | 53 | 2.30(b) |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the inspection programme established and implemented by the regulatory body for radioactive waste disposal facilities. [Topic 7.5] | B | F |
| Expectation | [Note: This primary question is only to reveal any specific information related to regulatory inspection as it applies to disposal facilities and activities. DO NOT REPEAT information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] The inspection programme of the regulatory body includes inspection of waste disposal facilities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 1 (Rev. 1) |  | 51 | Requirement 27, 28, 29 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the regulatory requirements for the development of different types of disposal facilities for radioactive waste? [Topic 9.5] | B | F |
| Expectation | There regulatory body has established requirements specific to each type of disposal facility for radioactive waste, including each type that is envisaged based, based on the national policy or legislation. The regulatory body has set conditions for the development, operation and closure of each individual disposal facility The regulatory body engages in dialogue with waste producers, the operators of the disposal facility and interested parties to ensure that the regulatory requirements are appropriate and practicable The regulatory body documents the procedures that it uses to evaluate the safety of each type of disposal facility, the procedures that operators are expected to follow in the context of licensing, important decisions prior to licensing and licence applications. The regulatory body has set out the procedures for meeting the requirements for the various stages of the licensing process. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 36 | Requirement 2 |
| Safety Standards Series No. SSR-5 |  | 37 | 3.9 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the requirements in place to carry out safety assessment and develop and maintain a safety case. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that establish that the operator of a disposal facility is responsible for its safety. There are regulatory requirements for the operator of a disposal facility to carry out all the necessary activities for site selection and evaluation, design, construction, operation, closure and if necessary, surveillance after closure. There are regulatory requirements to for the operator to establish technical specifications that are justified by safety assessment. There are regulatory requirements that the operation has to retain all information relevant to the safety case and the supporting safety assessment. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The operator has to be responsible for developing a disposal facility that is practicable and safe and for demonstrating its safety, consistent with the requirements of the regulatory body. This task has to be undertaken in consideration of: the characteristics and quantities of the radioactive waste to be disposed of; the site or sites available; the mining, excavation, construction and engineering techniques available; and the legal and regulatory infrastructure and regulatory requirements. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 37 | Requirement 3 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What are the requirements in place for preparation of a safety case, and for performing and updating of supporting safety assessment, as necessary, at each step of the development of a disposal facility? [Topic 9.5] | T | T |
| QData |  |  |  |
| Expectation | There are regulatory requirements to prepare a facility specific safety case early in the development of the disposal facility. There are regulatory requirements to perform safety assessment in support of the safety case and updated throughout the development and operation of the disposal facility |  |  |
| Help | The safety assessment has to identify key processes relevant to safety and to contribute to the development of an understanding of the performance of disposal facilities. It has to support judgements with regard to alternative management options as an element of optimizing protection and safety. Such an understanding has to provide the basis for the safety arguments presented in the safety case. The operator has to decide on the timing and the level of detail of the safety assessment, in consultation with, and subject to the approval of, the regulatory body |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 49 | Requirement 12 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What are the requirements related to the scope of a safety case for a disposal facility? [Topic 9.5] | T | T |
| QData |  |  |  |
| Expectation | There are regulatory requirements that the safety case describe all the safety relevant aspects of the site, the design of the facility, and the managerial control measures and regulatory controls. There are regulatory requirements that the safety case and supporting safety assessment demonstrate the level of protection of people and the environment provided There are regulatory requirements that the safety case and supporting safety assessment provide assurance that safety requirements will be met. |  |  |
| Help | With regard to safety after closure, the expected range of possible developments affecting the disposal system and events that might affect its performance, including those of low probability, have to be considered in the safety case and supporting assessment. The safety case may include the presentation of multiple lines of reasoning based, for example, on studies of natural analogues and palaeohydrogeological studies, suitable characteristics of the site, properties of the host geological formation, engineering considerations, operational procedures and institutional assurances. The performance of the disposal system under expected and less likely evolutions and events, which can be outside the design performance range of the disposal facility, has to be analysed in the safety assessment |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 50 | Requirement 13; para 4.17, 4.19 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the regulatory requirements in place for the operator to develop (and demonstrate) understanding of the relevance, and its implications for, safety of the available options for the facility throughout the process of development and operation of a disposal facility for radioactive waste. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that adequate level of confidence in the safety of each disposal facility has to be developed before decisions are taken There are regulatory requirements that the safety of the facility and the implications for safety of the available design and operational options for the disposal facility are considered and taken into account at each major decision point Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The understanding is for the purpose of providing an optimized level of safety in the operational stage after closure. Disposal facilities for radioactive waste may be developed and operated over a period of several years or several decades. Key decisions, such as decisions on site selection and evaluation, and on the design, construction, operation and closure of the disposal facility, are expected to be made as the project develops. In this process, decisions are made on the basis of the information available at the time, which may be either quantitative or qualitative, and the confidence that can be placed in that information |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 38 | Requirement 4; para. 3.19 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the regulatory requirements related to considerations when the disposal facility is sited, designed, constructed, operated and closed. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the site evaluation and the design, construction, operation and close of the disposal facility is in such a way that passive safety means are applied to the extent possible and that the need for actions to be taken after the closure of the facility is minimized. There are regulatory requirements to apply certain active control measures in the operational stage of a disposal facility for radioactive waste. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | To some extent, the safety of a disposal facility can depend on some future actions such as maintenance work or surveillance. However, this dependence has to be minimized to the extent possible. For a geological disposal facility, it is possible to provide for safety after closure by means of passive features. It is likewise possible to provide for the safety of a borehole disposal facility after closure by means of passive features, owing to the host geology. Providing for the safety of a disposal facility after closure by means of passive features will entail proper closure of the facility and ending the need for its active management. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 39 | Requirement 5 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the regulatory requirements related to developing an adequate understanding of the features of the disposal facility and its host environment and of the factors that influence its safety after closure over suitably long time periods (disposal system, safety case). [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements for the operator of a disposal facility to develop an adequate understanding of the features of the facility and its host environment and of the factors that influence its safety after closure over suitably long time periods (disposal system, safety case), so that a sufficient level of confidence in safety can be achieved. The regulatory body has recognized, during establishing the regulatory requirements, that there are various types and components of uncertainty inherent in modelling complex environmental systems, and that there are inevitably, significant uncertainties associated with projecting the performance of a disposal system over time. The regulatory body approves the range of possible events and processes causing disturbances that it is reasonable to include in the safety case. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Confidence has to be assured by the results of safety assessment for a disposal facility. The features of the facility and its host environment that provide for safety have to be identified, in addition to those factors that might be detrimental. It has to be demonstrated that these features and factors are sufficiently well characterized and understood. Any uncertainties have to be taken into consideration in the assessment of safety. The purpose of this demonstration is to establish, with a high level of confidence, that the disposal facility and its host environment can be relied on to provide the necessary containment and isolation over the timescales envisaged. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 40 | Requirement 6 |
| Safety Standards Series No. SSR-5 |  | 41 | 3.29 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the regulatory requirements to ensure safety by means of multiple safety functions throughout all the disposal facility lifetime, including the selection of the host environment, design of the engineered barriers of the facility and its operation. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the host environment be selected, the engineered barriers of the disposal facility be designed and the facility be operated to ensure that safety is provided by means of multiple safety functions. There are regulatory requirements that containment and isolation of the waste be provided by means of a number of physical barriers of the disposal system. There are regulatory requirements that the performance of these physical barriers be achieved by means of diverse physical and chemical processes together with various operational controls. There are requirements that the capability of the individual barriers and controls together with that of the overall disposal system to perform as assumed in the safety case be demonstrated. There are regulatory requirements that the overall performance of the disposal system shall not be unduly dependent on a single safety function. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The engineered and physical barriers that make up the disposal system are physical entities, such as the waste form, the packaging, the backfill, and the host environment and geological formation. A safety function may be provided by means of a physical or chemical property or process that contributes to containment and isolation, such as: impermeability to water; limited corrosion, dissolution, leach rate and solubility; retention of radionuclides; and retardation of radionuclide migration. Active controls can also fulfil safety functions or contribute to confidence in natural and engineered barriers and safety functions. The presence of a number of physical and other elements performing safety functions gives assurance that even if any of them do not perform fully as expected (e.g. owing to an unexpected process or an unlikely event), a sufficient margin of safety will remain. The physical elements and their safety functions can be complementary and can work in combination. The performance of a disposal system is thus dependent on different physical elements and on other elements that perform safety functions, which act over different time periods. For example, the roles of the waste package and the host geological formation for a geological disposal facility may vary in different time periods. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 42 | Requirement 7 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements related to the design of engineered barriers, including the waste form and packaging, and the selection of the host environment, so as to provide containment of the radionuclides associated with the waste? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the engineered barriers be designed and the host environment be selected so as to provide containment of the radionuclides associated with the waste. There are requirements that containment is provided until radioactive decay has significantly reduced the hazard posed by the waste. There are requirements that in the case of heat generating waste, containment be provided while the waste is still producing heat energy in amounts that could adversely affect the performance of the disposal system. The regulatory body ensures that the requirements for containment of radioactive waste on the early stages of facility planning/development (design) are complied with. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The containment of radioactive waste implies designing the disposal facility to avoid or minimize the release of radionuclides. Releases of small amounts of gaseous radionuclides and of small fractions of other highly mobile species from some types of radioactive waste may be inevitable. Such releases, nevertheless, have to be demonstrated to be acceptable by means of safety assessment. The containment may be provided by the characteristics of the waste form and the packaging and by the characteristics of other engineered components of the disposal system and the host environment and geological formation. The containment of the radionuclides in the waste form and the packaging over a defined period has to ensure that the majority of shorter-lived radionuclides decay in situ. For low level waste, such periods would be of the order of several hundred years; for high level waste the period would be several thousands of years. For high level waste, it also has to be ensured that any migration of radionuclides outside the disposal system would occur only after the heat produced by radioactive decay has substantially decreased. Radioactive waste from mining and mineral processing may include radionuclides with very long half-lives. Providing assurance of the integrity of the containment features of disposal facilities for such waste over the corresponding timescales requires particular consideration. If the waste has activity levels for which the dose and/or risk criteria for human intrusion into such facilities (see para. 2.15) might be exceeded, alternative disposal options will have to be considered. Possible alternative options include, for example, disposal of the waste below the surface, or separation of the radionuclide content giving rise to the higher dose, as determined by the safety case for the disposal facility. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 44 | Requirement 8 |
| Safety Standards Series No. SSR-5 |  | 54 | Requirement 16 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the requirements related to the siting, design and operation of the disposal facility. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that require the disposal facility to be sited, designed and operated to provide features that are aimed at isolation of the radioactive waste from people and from the accessible biosphere, including consideration of both the natural evolution of the disposal system and events causing disturbance of the facility. There are requirements that for near surface facilities, isolation be provided by the location and the design of the disposal facility and by operational and institutional controls. There are requirements that the disposal facility be located away from known areas of significant underground mineral resources or other valuable resources. There are requirements for assessment of the human intrusion possibility, in case it may not be possible to provide sufficient assurance of isolation from the biosphere Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | For geological disposal of radioactive waste, isolation is provided primarily by the host geological formation as a consequence of the depth of disposal. Isolation means design to keep the waste and its associated hazard apart from the accessible biosphere. It also means design to minimize the influence of factors that could reduce the integrity of the disposal facility. Sites and locations with higher hydraulic conductivities have to be avoided. Access to waste has to be made difficult to gain without, for example, violation of institutional controls for near surface disposal. Isolation also means providing for a very slow mobility of radionuclides to impede migration from disposal facilities. Location of a disposal facility in a stable geological formation provides protection of the facility from the effects of geomorphological processes, such as erosion and glaciation. Over time periods of several thousand years or more, the migration of a fraction of the longer lived and more mobile radionuclides from the waste in a geological disposal facility (or in other facilities that may include longer lived radionuclides, such as borehole facilities) may be inevitable. Caution needs to be exercised in applying safety criteria for periods far into the future. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 45 | Requirement 9; para. 3.43, 3.45 - 3.47 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the requirements related to applying an appropriate level of surveillance and control to protect and preserve the passive safety features, to the extent that is necessary. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements to apply an appropriate level of surveillance and control to protect and preserve the passive safety features to the extent that this is necessary so that they can fulfil the functions that they are assigned in the safety case for safety after closure. There are regulatory requirements that or geological disposal and for the disposal of intermediate level radioactive waste, the passive safety features (barriers) have to be sufficiently robust so as not to require repair or upgrading. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 46 | Requirement 10 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the requirements in place for disposal facilities to be developed, operated and closed in a series of steps. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that disposal facilities are developed, operated and closed in a series of steps, each supported, as necessary, by iterative evaluations of the site, of the options for design, construction, operation and management, and of the performance and safety of the disposal system. The regulatory body reviews the step-by-step development of a disposal facility, including the transparency and public involvement in these activities at each step of the process. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | A step-by-step approach to the development of a disposal facility for radioactive waste refers to the steps that are imposed by the regulatory body and by political decision-making processes. This approach is taken to provide an opportunity to ensure the quality of the technical programme and the associated decision making. For the operator, it provides a framework in which sufficient confidence in the technical feasibility and safety of the disposal facility can be built at each step in its development. The nature of the reviews and involvement will depend on national practices and on the facility in question. Technical reviews by, or on behalf of, the operator and the regulatory body may focus on site selection and evaluation and design options, the adequacy of the scientific basis and analyses, and whether safety standards and requirements have been met. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 47 | Requirement 11; para. 4.4 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in place related to the documentation of the safety case and supporting safety assessment for a disposal facility? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the safety case and supporting safety assessment for a disposal facility be documented to a level of detail and quality sufficient to inform and support the decision to be made at each step and to allow for independent review of the safety case and supporting safety assessment. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The necessary scope and structure of the documentation setting out the safety case and supporting safety assessment will depend on the step reached in the project for the disposal facility and on national requirements. This includes consideration of the needs of different interested parties for information. Important considerations in documenting the safety case and supporting safety assessment are justification, traceability and clarity |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 52 | Requirement 14; Para 4.25 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements related to the level of details in site characterization for a disposal facility? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the site for a disposal facility be characterized at a level of detail sufficient to support both a general understanding of the site characteristics and how the site will evolve overtime. There are regulatory requirements that site characterization should include its present condition, its probable natural evolution and possible natural events, and also human plans and actions in the vicinity that may affect the safety of the facility over the period of interest. There are regulatory requirements that site characterization should include a specific understanding of the impact on safety of features, events and processes associated with the site and the facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | An understanding of the site for a disposal facility has to be gained in order to present a convincing scientific description of the disposal system on which the more conceptual descriptions that are used in the safety assessment can be based. The focus has to be on features, events and processes relating to the site that could have an impact on safety and that are addressed in the safety case and supporting safety assessment. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 53 | Requirement 15, paras. 4.27 - 4.28 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the requirements in place for construction of a disposal facility in accordance with the design as described in the approved safety case and safety assessment. [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that the disposal facility be constructed in accordance with the design as described in the approved safety case and supporting safety assessment. There are requirements that the disposal facility be constructed in such a way as to preserve the safety functions of the host environment that have been shown by the safety case to be important for safety after closure. There are requirements that Construction activities be carried out in such a way as to ensure safety during the operational period. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area |  |  |
| Help | Construction of a disposal facility can be a complex technical undertaking that might be constrained, particularly if it is carried out underground, by the conditions and the properties of the host geological formation and by the techniques that are available for underground excavation and construction. An adequate level of characterization has to be completed before construction is begun. Excavation and construction activities have to be carried out in such a way as to avoid unnecessary disturbance of the host environment. Sufficient flexibility in engineering techniques has to be adopted to allow for variations to be encountered, such as variations in rock conditions or groundwater conditions in underground facilities. Excavation and construction of a disposal facility could continue after the commencement of operation of part of the facility and after the emplacement of waste packages. Such overlapping of construction and operational activities has to be planned and carried out so as to ensure safety, both in operation and after closure. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 55 | Requirement 17 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in place for a disposal facility to be operated in accordance with the conditions of the licence and the relevant regulatory requirements? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that a disposal facility be operated in accordance with the conditions of the licence and the relevant regulatory requirements to maintain safety during the operational period. There are requirements that a disposal facility be operated in such a manner as to preserve the safety functions assumed in the safety case that are important to safety after closure. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area |  |  |
| Help | All operations and activities important to the safety of a disposal facility have to be subjected to limitations and controls and emergency plans have to be put in place. The various procedures and plans have to be documented and the documentation has to be subject to appropriate control procedures. The safety case has to demonstrate that hazards and other radiation risks to workers and to members of the public under conditions of normal operation and anticipated operational occurrences have been reduced as low as reasonably achievable. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 56 | Requirement 18 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in relation to closure of a disposal facility? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that a disposal facility be closed in a way that provides for those safety functions that have been shown by the safety case to be important after closure. There are requirements that plans for closure, including the transition from active management of the facility, be well defined and practicable, so that closure can be carried out safely at an appropriate time. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 56 | Requirement 19 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What requirements are in place for waste packages and unpackaged waste accepted for emplacement in a disposal facility? [Topic 9.5] | B | F |
| Expectation | There are regulatory requirements that waste packages and unpackaged waste accepted for emplacement in a disposal facility conform to criteria fully consistent with and derived from the safety case for the operational and post-closure safety of the disposal facility. There are requirements that waste acceptance requirements and criteria for a given disposal facility have to ensure the fulfilment of the safety functions for the waste form and waste packaging with regard to safety in the long term. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Examples of possible parameters for waste acceptance criteria include the characteristics and performance requirements of the waste packages and the unpackaged waste to be disposed of, such as the radionuclide content or activity limits, the heat output and the properties of the waste form and packaging. Modelling and/or testing of the behaviour of waste forms has to be undertaken to ensure the physical and chemical stability of the different waste packages and unpackaged waste under the conditions expected in the disposal facility, and to ensure their adequate performance in the event of anticipated operational occurrences or accidents. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 57 | Requirement 20 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the regulatory requirements for a programme of monitoring that needs to be carried out prior to and during the construction and operation of a disposal facility, and after its closure. [Topic 9.5] | B | F |
| Expectation | There are requirements for programme of monitoring to be carried out prior to, and during, the construction and operation of a disposal facility and after its closure, if it is part of the safety case. There are requirements that the programme be designed to collect and update information necessary for the purposes of protection and safety. There are requirements that information be obtained to confirm the conditions necessary for the safety of workers and members of the public and protection of the environment during the period of operation of the facility. There are requirements that monitoring be carried out to confirm the absence of any conditions that could affect the safety of the facility after closure. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The purposes of the monitoring programme include: (a) Obtaining information for subsequent assessments; (b) Assurance of operational safety; (c) Assurance that conditions at the facility for operation are consistent with the safety assessment; (d) Confirmation that conditions are consistent with safety after closure Monitoring programmes have to be designed and implemented so as not to reduce the overall level of safety of the facility after closure. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 58 | Requirement 21 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What requirements are in place for plans that are prepared for the post-closure period to address institutional control and the arrangements for maintaining the availability of information on the disposal facility? [Topic 9.5] | B | F |
| Expectation | There are requirements that shall be prepared for the period after closure to address institutional control and the arrangements for maintaining the availability of information on the disposal facility. There are requirements that plans be consistent with passive safety features and form part of the safety case on which authorization to close the facility is granted. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The long-term safety of a disposal facility for radioactive waste has not to be dependent on active institutional control. Additionally, the safety of the disposal facility has not to be dependent solely on institutional controls. Institutional controls cannot be the sole or main component of safety for a near surface disposal facility. The ability of the institutional controls to provide the contributions to safety envisaged in the safety case has to be demonstrated and justified in the safety case |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 59 | Requirement 22 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in place for ensuring that safety is not compromised by the measures undertaken on accounting for, and control of, nuclear material in the design and operation of disposal facilities that are subjected to such agreements? [Topic 9.5] | B | F |
| Expectation | There are requirements that in the design and operation of disposal facilities subject to agreements on accounting for, and control of, nuclear material, consideration be given to ensuring that safety is not compromised by the measures required under the system of accounting for, and control of, nuclear material. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 61 | Requirement 23 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements in place for periodic safety assessment of existing disposal facilities? [Topic 9.5] | B | F |
| Expectation | There are requirements that the safety of existing disposal facilities to be assessed periodically until termination of the licence. There are requirements that safety also be assessed when a safety significant modification is planned or in the event of changes with regard to the conditions of the authorization. There are requirements that in the event that any new safety requirements are set down or are not met, measures be put in place to upgrade the safety of the facility. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. SSR-5 |  | 64 | Requirement 26 |