| Questionnaire Title | Facilities and Activities and Exposure Situations |
| --- | --- |
| Module Name | Safety Requirements for Public Exposure |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the authorization system of facilities and activities take into account the requirements for public exposure? [Topic 5.11] | B | F |
| Expectation | [Note: The part of this primary question related to Authorization is only to reveal any specific information related to authorization as it applies to public exposure. DO NOT REPEAT any information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] Requirements for control of public exposure established by legislation are taken into account during the authorization process. These requirements among others, are related to: - ensuring that protection and safety is optimized for planned public exposure; - dose constraints and dose limits specified for public exposures; - safety assessment addressing optimization of protection and safety, design criteria and features relating to the exposure; - protection against existing exposure situations; - approved operational limits and conditions relating to public exposures; - control of radioactive discharges and materials for clearance; - environmental monitoring for public radiation protection purposes. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 53 | Req. 3; 3.117 -3.144; 5.6-5.23 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the regulatory body take into account the requirements for public exposure during the review and assessment process for facilities and activities? [Topic 6.11] | B | F |
| Expectation | [Note: The part of this primary question related to Review and Assessment is only to reveal any specific information related to review and assessment as it applies to public exposure. DO NOT REPEAT any information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] Requirements for control of public exposure established by legislation and conditions of authorization are considered during the review and assessment process. The review and assessment process of the regulatory body for public exposure includes, among others: - Review and assessment of submitted documents related to potential public exposures of a new or modified practices; - Review and assessment of radiological environmental impact assessments related to facilities, activities or radiation sources; - Review of monitoring programmes of registrants and licensees; - Review of periodic reports on public exposures by registrants and licensees; - Verification of compliance of authorized practices with the respective requirements, - Assessment of total public exposure for different exposure situations; - Evaluation and assessment of reports of registrants and licensees on limit violations, incidents and accidents; - Evaluation of documents submitted by consumer product providers on demonstration of compliance with the requirements; - Evaluation of justification and optimization for protective or remedial actions and optimization of protection and safety in existing public exposure situations; - Periodical review of reference levels; - Review of the safety assessment of remedial action plans and their changes, of the related procedures, monitoring programmes, post-remedial control measures; - Evaluation of indoor radon concentration, exposure due to radioactivity in commodities and the related reference levels and planned actions. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 53 | Req. 3;3.117 -3.144;5.6-5.23 |
| Safety Standards Series No. GSR Part 3 |  | 93 |  |
| Safety Standards Series No. GSR Part 3 |  | 125 |  |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the regulatory body verify during inspection of facilities and activities compliance the requirements for public exposure? [Topic 7.11] | B | F |
| Expectation | [Note: The part of this primary question related to Inspection is only to reveal any specific information related to inspection as it applies to public exposure. DO NOT REPEAT any information that has already been provided in the core modules dealing with GSR Part 1 (Rev. 1).] Requirements for control of public exposure established by legislation and conditions of authorization are verified during inspection of facilities and activities. Inspection programme of the regulatory body includes inspection of registrants and licensees, of suppliers and providers of consumer products with regards to public exposures. Inspection of the regulatory body for protection of members of the public includes among others verification of: - compliance with the optimization requirements; - application of dose constraints and risk constraints; - compliance with the respective dose limits; - compliance with the operational limits and conditions related to public exposures; - safety measures and provisions preventing external exposures; - discharges to the environment by authorized parties; - remedial actions and protective actions in case of existing exposure situations. Inspection procedure(s)/guidance, check list(s) used during inspection of facilities and activities take into consideration public exposure requirements. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 53 | Requirement 3 |
| Safety Standards Series No. GSR Part 3 |  | 93 | 3.117 -3.144 |
| Safety Standards Series No. GSR Part 3 |  | 125 | 5.6-5.23 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the responsibilities of relevant parties and requirements established by the government or the regulatory body that are specific to public exposure. [Topic 9.11] | B | F |
| Expectation | The government or the regulatory body has established the responsibilities of registrants and licensees, of suppliers, and of providers of consumer products in relation to the application of requirements for public exposure in planned exposure situations. The government or the regulatory body has established and enforced requirements for the optimization of protection and safety for situations in which individuals are or could be subject to public exposure. The government or the regulatory body has established and enforces compliance with dose limits for public exposure specified in Schedule III of GSR Part 3. The regulatory body, before authorization of a new or modified practice, requires the submission of, and reviews, the safety assessments and other design related documents from the responsible parties that address the optimization of protection and safety, the design criteria and the design features relating to the assessment of exposure and potential exposure of members of the public. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Relevant parties includes registrants and licensees, suppliers and providers of consumer products (designers, manufacturers, producers, constructors, installers, distributors, sellers, and importers and exporters of consumer products). |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 93 | Requirement 29 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What factors are taken into account by the government or regulatory body for the establishment or approval of constraints on dose and constraints on risk to be used in optimization of protection and safety for members of the public? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | Constraints on source and risk have been established, taking into account, as appropriate: - The characteristics of the sources and of the practice with relevance for public exposure; - Good practice in the operation of similar sources; - Dose contributions from other authorized practices or from possible future authorized practices, estimated at the design and planning phase; - Views of interested parties. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 94 | 3.120 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | Describe the operational limits and conditions related to public exposure that the regulatory body has established or approves. [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body has established or approved operational limits and conditions, including authorized limits for discharges, that are used by registrants and licensees as the criteria for demonstration of compliance after the commencement of operation of a source. There are requirements that operational limits and conditions: - correspond to doses below the dose limits taking into account the results of optimization; - reflect good practice in the operation of similar facilities or activities. - allow for operational flexibility; - take into account the results of the prospective assessment for radiological environmental impacts that is undertaken in accordance with requirements of the regulatory body. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 94 | 3.123 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | When a source within a practice could cause public exposure outside the territory or other area under the jurisdiction or control of the State in which the source is located, what are the relevant arrangements in place for the government or the regulatory body? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | The government or the regulatory body ensures that the assessment for radiological impacts includes those impacts outside the territory or other area under the jurisdiction or control of the State. The government or the regulatory body, to the extent possible, establishes requirements for the control of discharges. The government or the regulatory body arrange with the affected State the means for the exchange of information and consultations, as appropriate. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 95 | 3.124 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the regulatory body ensure that relevant parties apply the system of protection and safety to protect members of the public against exposure? [Topic 9.11] | B | F |
| Expectation | The regulatory body verifies compliance of registrants, licensees, producers and suppliers of consumer products with the requirements of public exposure, taking into account: - Possible changes in any conditions that could affect exposure of members of the public, - Good practice in the operation of similar sources or the conduct of similar practices, - Possible build-up and accumulation in the environment of radioactive substances from discharges during the lifetime of the source, - Uncertainties in the assessment of doses, especially uncertainties in contributions to doses if the source and the representative person are separated in space or in time. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | Conditions that could affect exposure of members of the public could be such as changes in the characteristics and use of the source changes in environmental dispersion conditions, changes in exposure pathways or changes in values of parameters used for the determination of the representative person. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 95 | Requirement 30 and 3.126 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the government or regulatory body ensure that registrants and licensees, for sources under their responsibility, have established and maintained policies, procedures, plans and measures for public exposure? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are regulations/ regulatory requirements that require registrants and licensees to establish, implement and maintain, for sources under their responsibilities: - Policies, procedures and organizational arrangements for protection and safety in relation to public exposure. - Measures for ensuring optimization of protection and safety and limitation of exposure of members of the public from such sources, in accordance with the authorization: - Provision for suitable and adequate resources (including facilities, equipment and services) for the protection and safety of members of the public; - Programmes for appropriate training of personnel having functions relevant to protection and safety of members of the public, as well as periodic retraining as required. - Provision for appropriate monitoring equipment, monitoring programmes and methods for assessing public exposure. - Adequate records of monitoring programmes. - Emergency plans, emergency procedures and emergency arrangements, in accordance with the nature and magnitude of the associated radiation risks. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 96 | 3.127 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What requirements are in place for visitors to controlled and supervised areas? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are regulations/ regulatory requirements established in respect of public exposure for visitors to a controlled area or a supervised area. - to ensure that visitors are accompanied in any controlled area by a person who knows the measures for protection and safety for the controlled area; - to provide adequate information and instructions to visitors before they enter a controlled area or a supervised area; - to ensure that adequate control is maintained over the entry of visitors to a controlled area or a supervised area, including the use of signs for such areas. |  |  |
| Help | The objective of providing adequate information and instructions to visitors before they enter a controlled area or a supervised area is so as to provide for protection and safety for visitors and for other individuals who could be affected by their actions. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 96 | 3.128 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | Where a source can give rise to external or internal exposure of members of the public, what governmental or regulatory requirements are in place to ensure protection of public exposure? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are regulations/ regulatory requirements to ensure that members of the public are protected from external and internal exposure from radiation sources. There are regulatory requirements for registrants and licensees to ensure, if a source can give rise to external exposure of members of the public, that: - The floor plans and arrangements of equipment for all new installations utilizing such sources, as well as all significant modifications to existing installations, are subject, as appropriate, to review and approval by the regulatory body prior to commissioning; - Shielding and other measures for protection and safety, including access control, are provided as appropriate for restricting public exposure, in particular at open sites such as for some applications of industrial radiography. There are regulatory requirements for registrants and licensees to ensure, as appropriate, that: - Specific provisions for confinement are established for the design and - operation of a source that could cause the spread of contamination in areas - that are accessible to members of the public; - Measures for protection and safety are implemented for restricting public - exposure due to contamination in areas within a facility that are accessible - to members of the public. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 97 | 3.129 and 3.130 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the requirements for relevant parties to ensure that all discharges of radioactive material to the environment are managed in accordance with the authorization? [Topic 9.11] | B | F |
| Expectation | There are regulations/regulatory requirements for registrants and licensees, in applying for authorizations of discharges to ensure, as appropriate to: - determine the characteristics and activity of the material to be discharged, and the possible points and methods of discharge; - determine by an appropriate pre-operational study all significant exposure pathways by which discharged radionuclides could give rise to exposure of members of the public; - assess the doses to the representative person due to the planned discharges; - consider the radiological environmental impacts in an integrated manner with features of the system of protection and safety, as required by the regulatory body. There are regulations/regulatory requirements for registrants and licensees to ensure that control measures are reviewed and modified in light of operating experience or changes which may affect doses to the representative person. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 98 | Requirement 31 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the arrangements in place for the regulatory body and relevant parties to ensure that programmes for source monitoring and environmental monitoring are in place and that the results are recorded and made available? [Topic 9.11] | B | F |
| Expectation | There are regulatory requirements that registrants and licensees establish and implement monitoring programmes for public exposure due to sources under their responsibility and the programmes include monitoring of external exposure, discharges, radioactivity in the environment and other important parameters for assessment of public exposure. There are regulatory requirements for registrants and licensees to: - maintain appropriate records of results of monitoring and estimated doses to members of the public - report or make available to the regulatory body the results of the monitoring programme at approved intervals. - report promptly to the regulatory body any levels exceeding the operational - limits and conditions relating to public exposure, and any significant increase in dose rate or concentrations of radionuclides in the environment that could be attributed to the authorized practice. - establish and maintain a capability to conduct monitoring in an emergency in the event of unexpected increases in radiation levels or in concentrations of radionuclides in the environment. - verify the adequacy of the assumptions made for the assessment of public exposure and the assessment for radiological environmental impacts. The regulatory body reviews and approves the monitoring programmes of registrants and licensees. The regulatory body publishes or makes available on request, as appropriate, results from source monitoring and environmental monitoring programmes and assessments of doses from public exposure. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The report to the regulatory body of the results of the monitoring programme at approved intervals would include, as applicable, the levels and composition of discharges, dose rates at the site boundary and in premises open to members of the public, results of environmental monitoring and retrospective assessments of doses to the representative person. Reports to the regulatory body would be in accordance with reporting criteria established by the regulatory body. An event of unexpected increases in radiation levels or in concentrations of radionuclides in the environment could be due to an accident or other unusual event attributed to the authorized source or facility. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 99 | Requirement 32; para. 3.135; 3.136; 3.137 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the arrangements in place in relation to the use of consumer products that are made available to the public. [Topic 9.11] | B | F |
| Expectation | There are regulations/regulatory requirements that providers of consumer products ensure that consumer products are not made available to the public unless the justification of their use by members of the public has been approved by the government or the regulatory body, and either their use has been exempted on the basis of the criteria specified in Schedule I or their provision to the public has been authorized. There are regulations/regulatory requirements that ensure that registrants and licensees plan for appropriate arrangements for the servicing, maintenance, recycling or disposal of consumer products. Upon receipt of a request for authorization to provide consumer products to the public, the regulatory boy: - verifies the assessments and the selection of parameters presented in the request for authorization; - determines whether the end use of the consumer product can be exempted; - where appropriate, provides specific conditions of authorization. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 101 | Requirement 33 and para. 3.138 to 3.140 |
| Safety Standards Series No. SSG-36 |  | 28 | 2.1-2.39 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How is it ensured that the design and manufacture of consumer products are subject to optimization of protection and safety? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are regulations/regulatory requirements to ensure that designers, manufacturers and other providers of consumer products take into account - various radionuclides that could be used in consumer products and their radiation types, energies, activities and half-lives; - chemical and physical forms of the radionuclides that could be used in consumer products and their significance for protection and safety in normal conditions and abnormal conditions; - containment and shielding of radioactive substances in consumer products and access to these radioactive substances in normal conditions and abnormal conditions; - need for servicing or repair of consumer products and ways in which this could be done; - relevant experience with similar consumer products. There are regulations/regulatory requirements for providers of consumer products - to ensure that consumer products are labelled, and clear and appropriate information and instructions are provided with each consumer product, - to provide clear and appropriate information and instructions with each product, - to provide the consumer product retailers with appropriate information on safety and instruction on their transport and storage. |  |  |
| Help | Each consumer product should have proper labelling and information in accordance with paragraphs 3.142 (a)-(b) of GSR Part 3 The clear and appropriate information and instruction to be provided with each consumer product should be in accordance with paragraphs 3.143 (a)-(e) of GSR Part 3 |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 102 | 3.141- 3.144 |
| Safety Standards Series No. SSG-36 |  | 46 | 3.25–3.29 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the responsibilities of the government or the regulatory body regarding the public exposure in existing exposure situations? [Topic 6.11] | A | F |
| Expectation | The government has evaluated those existing exposure situations that have been identified to determine which occupational and public exposures are of concern from the point of view of radiation protection. For those existing exposure situations that have been identified, the government has assigned responsibilities for protection and safety e.g. regulatory body, public health authority etc. For those existing exposure situations that have been identified, reference levels have been established. The legal and regulatory framework includes provisions for the management of existing exposure situations that have been identified. The regulatory body or other national authority prepares and makes available information on the potential health risks of exposure to the existing exposure situation and on the means for the individuals to reduce their exposure and the associated health risks. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 123 | Req. 47 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Describe the provisions made by the government in its framework for protection and safety to identify those persons or organizations responsible for remediation of areas with residual radioactive material, and for putting in place an appropriate strategy for radioactive waste management arising from remedial actions. [Topic 6.11] | A | F |
| Expectation | For the remediation of areas with residual radioactive material deriving from past activities or from a nuclear or radiological emergency, the government has ensured that provision is made in the framework for protection and safety for: - The identification of those persons or organizations responsible for the contamination of areas and those responsible for financing the remediation programme, and the determination of appropriate arrangements for alternative sources of funding if such persons or organizations are no longer present or are unable to meet their liabilities; - The designation of persons or organizations responsible for planning, implementing and verifying the results of remedial actions; - The establishment of any restrictions on the use of or access to the areas concerned before, during and, if necessary, after remediation; - An appropriate system for maintaining, retrieval and amendment of records that cover the nature and the extent of contamination; the decisions made before, during and after remediation; and - System for maintaining, retrieval and amendment of information on verification of the results of remedial actions, including the results of all monitoring programmers after completion of the remedial actions. - A strategy for radioactive waste management is put in place to deal with any waste arising from the remedial actions Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.10 and 5.11 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | Describe the regulatory requirements in place for the persons or organizations responsible for the planning, implementation and verification of remedial actions. [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are requirements that the persons or organizations responsible for the planning, implementation and verification of remedial actions, as appropriate, ensure that: - A remedial action plan, supported by a safety assessment, is prepared and is submitted to the regulatory body or other relevant authority for approval. - The remedial action plan is aimed at the timely and progressive reduction of the radiation risks and eventually, if possible, at the removal of restrictions on the use of or access to the area. - Any additional doses received by members of the public as a result of the remedial actions are justified on the basis of the resulting net benefit, including consideration of the consequent reduction of the annual dose. - Procedures are in place for reporting to the regulatory body or other relevant authority on any abnormal conditions relevant to protection and safety. |  |  |
| Help | In the choice of the optimized remediation option: - Radiological impacts on people and the environment are considered together with non-radiological impacts on people and the environment, and with technical, societal and economic factors; - The costs of the transport and management of radioactive waste, the radiation exposure of and health risks to the workers managing the radioactive waste, and any subsequent public exposure associated with its disposal are all taken into account. - A mechanism for public information is in place and interested parties are involved in the planning, implementation and verification of the remedial actions, including any monitoring following remediation. - A monitoring programme is established and implemented. - A system for maintaining adequate records relating to the existing exposure situation and to actions taken for protection and safety is in place. |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.12 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | Describe the responsibilities of the regulatory body, or other relevant authority, in relation to remediation of areas with residual radioactive material. [Topic 6.11] | T | T |
| QData |  |  |  |
| Expectation | The regulatory body or other relevant authority takes responsibility, in particular for: - Establishing and implementing a protection strategy; - Establishing appropriate reference levels for existing exposure situation - Reviewing of the safety assessment submitted by the responsible person or organization, approval of the remedial action plan and of any subsequent changes to the remedial action plan, and granting of any necessary authorization; - Establishing of criteria and methods for assessing safety; - Reviewing of work procedures, monitoring programmes and records; - Reviewing and approval of significant changes to procedures or equipment that may have radiological environmental impacts or that may alter the exposure conditions for workers taking remedial actions or for members of the public; - Where necessary, establishing of regulatory requirements for control measures following remediation. |  |  |
| Help | The regulatory body or other relevant authority takes responsibility in accordance with para. 2.29 and 5.3, 5.4 and 5.5 of GSR Part 3 |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.13 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What responsibilities are assigned to the person or organization responsible for carrying out remedial actions? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are requirements that the person or organization responsible for carrying out the remedial actions: - ensures that the work, including management of the radioactive waste arising, is conducted in accordance with the remedial action plan; - takes responsibility for all aspects of protection and safety, including the conduct of a safety assessment; - monitors the area regularly during the remediation so as to verify levels of contamination, to verify compliance with the requirements for radioactive waste management, and to enable any unexpected levels of radiation to be detected and the remedial action plan to be modified accordingly, subject to approval by the regulatory body or other relevant authority; - performs a radiological survey after completion of remedial actions to demonstrate that the end point conditions, as established in the remedial action plan, have been met; - prepares and retains a final remediation report and shall submit a copy to the regulatory body or other relevant authority. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.14 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | What are the actions taken by the regulatory body or other relevant authority after the remedial actions have completed? [Topic 6.11] | T | T |
| QData |  |  |  |
| Expectation | After the remedial actions have been completed, the regulatory body or other relevant authority: - reviews, amend as necessary and formalize the type, extent and duration of any post-remediation control measures already identified in the remedial action plan, with due consideration of the residual radiation risks. - identifies the person or organization responsible for any post-remediation control measures. - where necessary, imposes specific restrictions for the remediated area to control: - Access by unauthorized persons; - Removal of radioactive material or use of such material, including its use in commodities; - Future use of the area, including the use of water resources and its use for the production of food or feed, and the consumption of food from the area. - periodically reviews conditions in the remediated area and, if appropriate, shall amend or remove any restrictions. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.15 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | Describe the requirements in place for post-remediation control measures. [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | There are regulatory requirements that require that person or organization responsible for post-remediation control measures establish and maintain, for as long as required by the regulatory body or other relevant authority, an appropriate programme, including any necessary provision for monitoring, to verify the long-term effectiveness of the completed remedial actions for areas in which controls are required after remediation. There are regulatory requirements that the conditions prevailing after the completion of remedial actions, if the regulatory body or other relevant authority has imposed no restrictions or controls, be considered to constitute the background conditions for any new facilities and activities or for habitation on the land. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.16; 5018 |
| Safety Standards Series No. GSG-15 |  | 107 | 10.1-10.5 |

| QTYPE | Subsidiary Question | True/False | Compulsory |
| --- | --- | --- | --- |
| T | How does the government, in consultation with interested parties, ensure that arrangements are in place, as necessary, for the continuing control of exposure with the aim of establishing conditions for sustainable living? [Topic 9.11] | T | T |
| QData |  |  |  |
| Expectation | The government in consultation with interested parties ensures, for those areas with long lasting residual radioactive material, in which the government has decided to allow habitation and the resumption of social and economic activities, that arrangements are in place, as necessary, for the continuing control of exposure with the aim of establishing conditions for sustainable living, including: - Establishment of reference levels for protection and safety that are consistent with day-to-day life; - Establishment of an infrastructure to support continuing ‘self-help protective actions’ in the affected areas, such as by the provision of information and advice, and by monitoring. |  |  |
| Help |  |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 126 | Requirement 49 and para. 5.17 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | How does the government provide information on exposure due to radon and the associated health risks, including the increased risks due to smoking? [Topic 9.11] | A | F |
| Expectation | The government has prepared information on exposure due to radon and the associated health risks, including increased health risks due to smoking, and is provided to members of the public and other interested parties. The government (through the regulatory body or other relevant authority) collects available information on the levels of radon indoors i.e. in dwellings and other buildings of high occupancy for members of the public. The information of radon levels could include: - The name of the organization that collected the data on radon; - The number of dwellings surveyed; - Whether the measurements are representative of the country (are they national, particular regions etc); - The average level of radon in the country or region (in Bq/m3); - The range of radon levels in the country. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help |  |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 129 | Req. 50; 5.19(a)(b) |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | Where the radon levels are of concern for public health, what are the arrangements by the government (through the regulatory body or other relevant authority) in place to control public exposure due to radon indoors? [Topic 6.11] | A | F |
| Expectation | The government has evaluated the data on radon levels in the country and has decided to establish an action plan to control public exposure to radon indoors. The government considers that the measures in the radon action plan are justified in that the corrective actions and preventive measures to reduce radon levels in buildings are expected to yield sufficient benefits to outweigh the detriments associated with taking them. The radon action plan in included in the legal and regulatory framework. The action plan is implemented and coordinated by the regulatory body or other relevant authority e.g. public health authority. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area. |  |  |
| Help | The need for including in a national action plan measures for the control of public exposure to Rn-220 should be consistent with the public health priorities of the State. The action plan to control public exposure to radon should include the following components: - Assignment of responsibilities; coordination mechanisms; - Strategy for communication to increase public awareness of radon levels and the associated health risk, including in relation to smoking - Measurements of radon in dwellings, including in a national survey of radon in dwellings; measurement protocols; how results are reported; quality control programme for radon measurements; training/accreditation of experts making measurements; management of data; development of radon maps; etc. - Basis for setting a national reference level for radon in dwellings and other buildings of high occupancy by the public - Defining radon prone areas - Techniques for the control and reduction of radon in existing buildings (corrective actions). Evaluation of the effectiveness of the techniques. - Information is provided to the public on such measures. Requirements for measurement of radon at the time of sale of buildings. - Preventive measures to control radon levels in new buildings e.g. through building codes for new dwellings in radon prone areas. Measurement of radon is required in new buildings in radon prone areas. Issues of certificate of occupancy. - Training programme for building professionals. Certification system for companies offering corrective actions/preventive measures. - Evaluation of effectiveness of the radon reduction strategy; - Review of the radon action plan; - If radon levels of public health concern have been identified, measures for the control of public exposure to radon. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 129 | Req. 50 |

| QTYPE | Primary Question | SA | IL |
| --- | --- | --- | --- |
| Boolean | What are the reference levels established by the regulatory body (or other relevant authority) for radionuclides in commodities, such as construction materials, food, drinking water? [Topic 9.11] | A | F |
| Expectation | The regulatory body or other relevant authority establishes specific reference levels for exposure due to radionuclides in commodities such as construction materials, food and feed, and in drinking water, each of which are typically expressed as, or be based on, an annual effective dose to the representative person that generally does not exceed a value of about 1 mSv. The control of one or more of these commodities is included in the legal and regulatory framework. An authority has been designated as being responsible for the establishment and implementation of protection strategies for one or more of these commodities. Answer should include description of any measures planned to improve compliance with IAEA safety requirements relevant to this area |  |  |
| Help | The regulatory body or other relevant authority considers the guideline levels for radionuclides in food traded internationally that could contain radioactive substances as a result of a nuclear or radiological emergency, which have been published by the Joint Food and Agriculture Organization of the United Nations/World Health Organization Codex Alimentarius Commission. The regulatory body or other relevant authority also considers the guideline levels for radionuclides contained in drinking water that have been published by the World Health Organization. |  |  |
| Question Tags (one row per Tag) |  |  |  |
| Category | Name |  |  |
| References (one row per reference) |  |  |  |
| Name | Comments | Page | Para |
| Safety Standards Series No. GSR Part 3 |  | 131 | Req. 51 |